Application No.:  364181

Equipment Category – Asphalt Batch Plant

| 1. GENERAL INFORMATION |  |  | DATE: 11/14/2003 |
| --- | --- | --- | --- |
| a. manufacturer: |  |  |  |
| b. type: |  | c. model: |  |
| d. style: |  |  |  |
| e. applicable aqmd rules: 401, 402, 403, 404, 405 |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: 8 hrs/day 5 days/wk 50 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 364181 |
| --- | --- | --- | --- |
| A. FUNCTION: Production of hot-mix asphalt. Recycled asphalt is mixed with aggregate (sand, gravel or crushed rock) and, in some cases, ground rubber and/or mineral filler (mainly lime) and heated and dried in a rotary dryer. Liquid asphalt is added to this mix to produce asphaltic concrete. |  |  |  |
| B. maximum Heat Input: 175 MMBtu/hr |  | C. maximum throughput: 600 tph |  |
| D. burner information: no.: 1 type: LNB |  |  |  |
| e. primary fuel: Natural Gas |  | f. other fuel: None |  |
| g. operating conditions: Steady |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 364181 |  |
| --- | --- | --- | --- | --- |
| A. NAME: Blue Diamond Division of Sully Miller Contracting |  |  |  | B. SIC CODE: 2951 |
| C. ADDRESS: 5625 Southern Ave. CITY: South Gate STATE: CA ZIP: 90280 |  |  |  |  |
| D. CONTACT PERSON: Pete Pouwels |  | E. PHONE NO.: 714-449-2288 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: 364181 |
| --- | --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Hiram Fong |  |  | D. PHONE NO.: 909-396-2718 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: 364181 ISSUANCE DATE: 10/6/2000 check if no p/c P/O NO.: F55717 ISSUANCE DATE: 10/16/2002 |  |  |  |  |
| F. START-UP DATE: September 2001) |  |  |  |  |

| 5. EMISSION INFORMATION |  |  | App. No.: 364181 |  |  |
| --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |
| A1. PERMIT LIMIT: The rotary dryer and the rubber and mineral storage silos load-out chutes to be vented to a baghouse. Aggregate charged and in process to be kept sufficiently moist to prevent visible dust emissions. Rotary dryer to be equipped with low-NOx burner and flue gas recirculation. NOx from the dryer not to exceed 33 ppmvd@3%O2. Asphalt storage tanks to be vented through condensers and steel wool filters with condenser-exit temperatures not to exceed 120F. Rubber and mineral filler storage silos to have filtered vents and pneumatic fill tubes. Truck load-out areas to be enclosed tunnels vented to two-stage filter packs. |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: Same as above |  |  |  |  |  |
| a3. BASIS OF THE BACT/LAER DETERMINATION: The above BACT determination was based on AQMD BACT Guidelines, Part D. Control of NOx to 33 ppmvd@3%O2 exceeds the AQMD BACT Guideline and is now achieved in practice. |  |  |  |  |  |
| ##### B. CONTROL TECHNOLOGY |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Baghouse (permitted separately, A/N 364182): Roto-Aire 420-S, reverse air, with 400 hp blower, 120,000 cfm. Low-NOx burner: Hauck, model ES-175X. Load-out tunnel filters: first stage-Farr 30/30 pleated filters, second stage-Riga Flow XL pleated filters. |  |  |  |  |  |
| B2. TYPE: |  |  |  |  |  |
| B3. DESCRIPTION: |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: 364182 ISSUANCE DATE: 10/6/2000 P/O NO.: F55718 ISSUANCE DATE: 10/16/2002 |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: 120,000 cfm ACTUAL CONTAMINANT LOADING: BLOWER HP: 400 |  |  |  |  |  |
| B6. WARRANTY: |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: NOx, CO, VOC, PM10 |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: None |  |  |  |  |  |
| B9. SPACE REQUIREMENT: |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  | ###### B11. UNUSED |  |
| B12. OPERATING HISTORY: Plant has been in normal operation since startup. There have been no problems with the low-NOx burner or flue gas recirculation system. |  |  |  |  |  |
| ### B13. UNUSED |  | ### B14. UNUSED |  |  |  |
| ##### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in EQUIPMENT cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: Lalo Bakhoum DATE: 5/9/02, 8/27/02, 5/27/03 |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: Inspections |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: None DATES: CAUSES: |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: None DATES: CAUSES: |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: Feb 20, 21, 27, 2002; Mar 11, 2002 CAPTURE EFFICIENCY: DESTRUCTION EFFICIENCY: OVERALL EFFICIENCY: SOURCE TEST/PERFORMANCE DATA: Virgin Aggregate Recycled Asphalt Product (RAP) Feb 2002 March 2002 Feb 20 Feb 21 Feb 27 Avg. Test 1 Test 2 Test 3 Avg. O2, % (dry) 8.59 8.90 7.83 8.44 7.1 7.5 6.8 7.1 NOx, ppmvd@3%O@ 23.5 26.7 27.4 25.9 26 25 24 25 CO, ppmvd@3%O2 35.9 38.1 44.3 39.4 91 41 108 80 PM, grn/dscf Probe + Filter .000125 .000581 .000380 .000362 .0035 .0014 .0031 .0027 Total .00128 .00126 .00348 .00201 .012 .031 .010 .018 VOC, ppmvd as CH4 6.2 -- -- 6.2 13.6 11.6 -- 12.6 OPERATING CONDITIONS: February: 472 to 486 tph; March: 500 tph, 15% RAP (avg.) TEST METHODS: AQMD Method 100.1 for NOx, CO and O2. AQMD Method 25.3 for VOC. For PM: AQMD Method 5.2 in Feb and EPA Method 5 in March. Three one-hour tests in each case. Test was approved by permitting team. |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: 364181 |
| --- | --- | --- |