Application No.:	  344221

Equipment Category – Catalyst Regeneration - Fluidized Catalytic Cracking Unit

| 1. general information |  |  | DATE: 8/21/2003 |
| --- | --- | --- | --- |
| a. manufacturer: |  |  |  |
| b. type: |  | c. model: |  |
| d. style: |  |  |  |
| e. applicable aqmd rules: 1105, Reg. XX-RECLAIM |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: 24 hrs/day 7 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 344221 |
| --- | --- | --- | --- |
| A. FUNCTION: The fluidized catalytic cracking unit (FCCU) converts heavy refinery products to lighter products that can be used as gasoline blendstocks and feedstocks to alkylation. There is a catalyst regeneration section, in which coked catalyst is regenerated by burning off the carbon. Gas from this section contains high levels of carbon monoxide (CO). The exit gas from this section goes to the air pollution control system (described below in Section 5B), then passes through an unfired waste heat recovery boiler to a stack. |  |  |  |
| b. size/dimension/capacity: 21'D x 108'H fractionator main column; 37'-2"D x 45'H regenerator |  |  |  |
| c. blowers: |  | d. total flow rate: scfm |  |
| e. material stored/processed/handled: Heavy refinery products |  |  |  |
| f. throughput/process rate/usage rate: |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 344221 |  |
| --- | --- | --- | --- | --- |
| A. NAME: ExxonMobil Corporation |  |  |  | B. SIC CODE: 2911 |
| C. ADDRESS: 3700 W. 190th Street CITY: Torrance STATE: CA ZIP: 90509 |  |  |  |  |
| D. CONTACT PERSON: Edward Filadelfia |  | E. PHONE NO.: 310-212-1826 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: 344221 |
| --- | --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Khang Nguyen |  |  | D. PHONE NO.: 909-396-3210 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: 344221 ISSUANCE DATE: 8/3/1999 check if no p/c P/O NO.: ISSUANCE DATE: |  |  |  |  |
| F. START-UP DATE: April-May 2000 |  |  |  |  |

| 5. EMISSION INFORMATION |  |  |  | App. No.: 344221 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |  |
| A1. PERMIT LIMIT: RECLAIM emission rates: NOx from CO boiler-.059 lb/MMBtu, SOx from CO boiler-0.83 lb/MMSCF natural gas and 58.25 lb/MMCF refinery gas. |  |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: This permit modification was for the purpose of adding a NOx control system and was not subject to BACT/LAER. The NOx control technology that was added, which is described below, is now achieved in practice for this equipment category. |  |  |  |  |  |  |
| A3. BASIS OF THE BACT DETERMINATION: Achieved in practice |  |  |  |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Mitshubishi |  |  |  |  |  |  |
| B2. TYPE: Selective Catalytic Reduction (SCR) of NOx |  |  |  |  |  |  |
| B3. DESCRIPTION: Gases from the FCCU regenerator section pass to a CO boiler, in which sensible heat is recovered and the CO is oxidized to CO2. The CO boiler is equipped with four low-NOx burners for a total capability of 464 MMBtu/hr supplemental firing of natural or refinery gas. The boiler is equipped with a urea injection system for NOx control (NOxOUT process). Flue gas from the CO boiler flows to an electrostatic precipitator (ESP), then to the SCR NOx reduction unit, then to an unfired waste heat boiler and finally to a stack. The CO boiler and ESP represent conventional control technologies for FCCUs. The NOxOUT process was installed in 1988 for additional NOx control and was new technology at the time. SCR technology is well known but has heretofore not been applied to FCCUs in this country. The SCR unit consists of a "honeycomb" type vanadium pentoxide catalyst bed in which ammonia reacts selectively with NOx to form N2, O2 and H2O. Ammonia injection is normally required; however, in this case, sufficient ammonia is already being injected upstream of the ESP for fly ash conditioning. The SCR not only reduces NOx but also reduces emissions of ammonia. As part of the SCR retrofit, the ammonia injection nozzles were improved by adding carrier air to enhance ammonia distribution in the flue gas. |  |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: 344222 ISSUANCE DATE: 8/3/1999 P/O NO.: ISSUANCE DATE: |  |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: 300,000 scfm (design) ACTUAL CONTAMINANT LOADING: Design NOx level is 55 ppmvd. BLOWER HP: Increased from 1000 to 3000 for SCR retrofit. |  |  |  |  |  |  |
| B6. WARRANTY: |  |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: NOx, CO, SOx, PM, VOC, SOx |  |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: NH3 |  |  |  |  |  |  |
| B9. SPACE REQUIREMENT: SCR reactor dimensions are 56'H x 31'L x 27'W |  |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  |  |  | B11. UNUSED |
| B12. OPERATING HISTORY: The refinery reports that the SCR unit has operated well and has achieved its design NOx removal since startup and de-bugging activities concluded at the end of 2000. No scheduled maintenance has yet been performed. The unit was inspected once after approximately a year of service and was in good condition. Accumulated dust was vacuumed out of the catalyst bed at that time. |  |  |  |  |  |  |
| B13. UNUSED |  |  | B14. UNUSED |  |  |  |
| ### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in capital cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: |  |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: 1 DATES: December 2001 CAUSES: ID fan control problem |  |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: 1 DATES: 11/30/2001 CAUSES: Venting from SCR bypass stacks |  |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |  |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: April and October 2002 CAPTURE EFFICIENCY: DESTRUCTION EFFICIENCY: OVERALL EFFICIENCY: SOURCE TEST/PERFORMANCE DATA: April 2-4, 2002 October 17-18, 2002 Flue Gas, dscfm 230,100 239,900 O2, % (dry) 3.17 2.79 NOx-inlet, ppmvd 77.6 112 NOx-outlet, ppmvd 4.53 10.6 NOx Removal, % 94 91 NH3-inlet, ppmvd 163 356 NH3-outlet, ppmvd 67.1 171 OPERATING CONDITIONS: Normal TEST METHODS: These are the latest two CEMS relative accuracy test audits (RATAs). NOx/SO2 RATAs are done every 6 months. The RATAs are done both upstream and downstream of the SCR due to presence of two bypass stacks at the upstream location, which are equipped with a CEMS. Ammonia is measured concurrently with each RATA. Measurement methods are as follows. AQMD Method 100.1 is used for NOx and O2--multi-point sampling, triplicate. AQMD Method 207.1 is used for NH3--single-point sampling, single 1-hr test. (Ammonia measurements were performed on 5 consecutive days in the October test session, and inlet and outlet values were found to vary widely day to day.) AQMD Methods 1 through 4 are used for dry gas flow. |  |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: 344221 |
| --- | --- | --- |
| The Permit to Operate for this modification is being processed together with other permit modifications and, for this reason, has not yet been issued. In this case, the SCR catalyst seems to be holding up well without any maintenance activities being required so far. However, in general, SCR catalyst maintenance may be required more frequently than the normal FCCU maintenance outage schedule, and a bypass may be needed in many cases to permit SCR catalyst maintenance to be performed without shutting down the FCCU. |  |  |