Application No.:	  394320

Equipment Category – Fiber Impregnation

| 1. general information |  |  | DATE: 1/30/2004 |
| --- | --- | --- | --- |
| a. manufacturer: Faustel/Despatch |  |  |  |
| b. type: Dip Pan with Oven |  | c. model: |  |
| d. style: Multi-Zone Oven with Dip Pan Underneath |  |  |  |
| e. applicable aqmd rules: 1128, 1171 |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: 24 hrs/day 7 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 394320 |
| --- | --- | --- | --- |
| A. FUNCTION: Fiberglass cloth web is continuously drawn through a liquid resin/solvent dip pan and an oven. Resin-impregnated cloth, "pre-preg", is used to manufacture inner layer panels of printed circuit boards. |  |  |  |
| b. size/dimension/capacity: Three 200,000 Btu/hr indirect gas heated zones and one 70.2 kW electrically heated zone |  |  |  |
| c. blowers: Five Process Air Blowers (two 10 hp, one 5 hp and two 3 hp) and three 10 hp radiant duct blowers. |  | d. total flow rate: 7500 scfm |  |
| e. material stored/processed/handled: |  |  |  |
| f. throughput/process rate/usage rate: |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 394320 |  |
| --- | --- | --- | --- | --- |
| A. NAME: Nelco Products, Inc. |  |  |  | B. SIC CODE: 3679 |
| C. ADDRESS: 1411 East Orangethorpe Ave. CITY: Fullerton STATE: CA ZIP: 92831 |  |  |  |  |
| D. CONTACT PERSON: Sam Khalil |  | E. PHONE NO.: 714-870-2210 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: 394320 |
| --- | --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Todd Iwata |  |  | D. PHONE NO.: 909-396-2574 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: ISSUANCE DATE: check if no p/c P/O NO.: F53610 ISSUANCE DATE: 7/17/2000 |  |  |  |  |
| F. START-UP DATE: |  |  |  |  |

| 5. EMISSION INFORMATION |  |  |  | App. No.: 394320 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |  |
| A1. PERMIT LIMIT: Facility-wide VOC cap of 150 lb/day. Compliance with Rules 1171 (compliance is by use of acetone for cleanup) and 1128 (minimum VOC collection and destruction efficiencies of 90% and 95%, respectively). System to be vented to thermal oxidizer operated with minimum chamber temperature of 1400F. Minimum 98% overall VOC control efficiency. |  |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: 98% overall VOC control efficiency |  |  |  |  |  |  |
| A3. BASIS OF THE BACT DETERMINATION: Achieved in Practice |  |  |  |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: MMT Environmental Services |  |  |  |  |  |  |
| B2. TYPE: Permanent Total Enclosure (oven and dip pan housing) vented to Thermal Oxidizer |  |  |  |  |  |  |
| B3. DESCRIPTION: Thermal Oxidizer Model No. Sigma-7550-50. Maximum thermal input 5.2 MMBtu/hr. Thermal oxidizer has recuperative heat exchanger and secondary heat exchanger used to preheat process air used to sweep the oven zones. |  |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: ISSUANCE DATE: P/O NO.: F56976 ISSUANCE DATE: 12/12/2002 |  |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: 7500 scfm ACTUAL CONTAMINANT LOADING: 1% (vol.) as C1 BLOWER HP: 60 |  |  |  |  |  |  |
| B6. WARRANTY: |  |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: VOC |  |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: NOx, CO, PM10 |  |  |  |  |  |  |
| B9. SPACE REQUIREMENT: |  |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  |  |  | B11. UNUSED |
| B12. OPERATING HISTORY: |  |  |  |  |  |  |
| B13. UNUSED |  |  | B14. UNUSED |  |  |  |
| ### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in capital cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: Eleanor Eleazar DATE: 10/8/2002 |  |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: Inspection |  |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: NONE DATES: CAUSES: |  |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: NONE DATES: CAUSES: |  |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |  |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: 11/6/2002 CAPTURE EFFICIENCY: 100 DESTRUCTION EFFICIENCY: 99.4 OVERALL EFFICIENCY: 99.4 SOURCE TEST/PERFORMANCE DATA: Inlet Outlet Temperature, F 378 669 Flow Rate, dscfm 7,260 6,990 O2, % (dry) 19.7 16.9 CO2, % (dry) 1.2 2.8 NMHC, ppmvd as C1 10,002 59 NMHC, lb/hr as C 138 0.78 Destruction Efficiency, % 99.4 OPERATING CONDITIONS: Chamber temperature: 1453F. TEST METHODS: AQMD Methods 1.1 (Multi-Point Traverse), 25.1 (for inlet VOC), 25.3 (for outlet VOC). Concurrent inlet/outlet sampling, duplicate tests. USEPA Method 204 used to certify permanent total enclosure. Test report approved by AQMD's Source Test Engineering group. |  |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: 394320 |
| --- | --- | --- |
| The fiberglass impregnation system and thermal oxidizer were each granted a Permit to Construct 11/29/2000 under application numbers 372871 and 372872, respectively. After some operation experience, it was discovered that concentrations of hydrocarbons in some areas of the system were near or above explosion limits; and application numbers 394320 and 394321 were received in December 2001 for modification of the air blowers to increase the amount of air passing through the system. After completion of modifications, a source test was performed; however the the 98% overall VOC control efficiency was not achieved. Application number 408965 was received in November 2002 to increase the maximum heat input to the oxidizer. This corrected the problem and the unit was retested (results shown above). |  |  |