Application No.:	  321072

Equipment Category - Flexographic Printing

| 1. general information |  |  | DATE: 11/16/2000 |
| --- | --- | --- | --- |
| a. manufacturer: Chadwick |  |  |  |
| b. type: Flexographic with Oven |  | c. model: P-599 |  |
| d. style: |  |  |  |
| e. applicable aqmd regulation xi rules: 1130, 1171 |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: 24 hrs/day 7 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 321072 |
| --- | --- | --- | --- |
| A. FUNCTION: This equipment is used to print on plastic bags for a variety of end uses. |  |  |  |
| b. size/dimension/capacity: Six color printing press, 48" web, with a 200,000 btu/hr natural gas fired oven |  |  |  |
| c. blowers: |  | d. total flow rate: scfm |  |
| e. material stored/processed/handled: |  |  |  |
| f. throughput/process rate/usage rate: |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 321072 |  |
| --- | --- | --- | --- | --- |
| A. NAME: Poly Pak America Inc. |  |  |  | B. SIC CODE: 2752 |
| C. ADDRESS: 2939 E. Washington Blvd CITY: Los Angeles STATE: CA ZIP: 90023-4277 |  |  |  |  |
| D. CONTACT PERSON: John Jakositz |  | E. PHONE NO.: (323) 264-2400 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: 321072 |
| --- | --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Todd Iwata |  |  | D. PHONE NO.: (909)396-2574 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: ISSUANCE DATE: check if no p/c P/O NO.: F28504 ISSUANCE DATE: 4/28/00 |  |  |  |  |
| F. START-UP DATE: 1994 |  |  |  |  |

| 5. EMISSION INFORMATION |  |  |  | App. No.: 321072 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |  |
| A1. PERMIT LIMIT: 1) This equipment shall comply with Rule 1130 and 1171; 2) VOC emissions from this facility shall not exceed 265 lbs in any one day; 3) VOC emissions from this equipment shall be vented to air pollution control equipment that has an overall efficiency of 95% |  |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: Permanent total enclosure and regenerative thermal oxidizer at 95% overall control efficiency. |  |  |  |  |  |  |
| A3. BASIS OF THE BACT DETERMINATION: Proposed by applicant |  |  |  |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Ship & Shore |  |  |  |  |  |  |
| B2. TYPE: |  |  |  |  |  |  |
| B3. DESCRIPTION: Regenerative thermal oxidizer, Model No. AMJ-10K-95, 3.4 mmbtu/hr natural gas fired |  |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: 323930 ISSUANCE DATE: 2/10/1997 P/O NO.: F29208 ISSUANCE DATE: 5/4/2000 |  |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: 10,000 scfm ACTUAL CONTAMINANT LOADING: BLOWER HP: 50 HP |  |  |  |  |  |  |
| B6. WARRANTY: |  |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: VOC (Press and Oven), and NOx, SOx, PM10, CO (Oven) |  |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: |  |  |  |  |  |  |
| B9. SPACE REQUIREMENT: |  |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  |  |  | B11. UNUSED |
| B12. OPERATING HISTORY: |  |  |  |  |  |  |
| B13. UNUSED |  |  | B14. UNUSED |  |  |  |
| ### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in capital cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: S.Ebiner, T.Iwata INSPECTOR'S NAME: DATE: 12/28/1999 |  |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: Record Keeping and Source Testing |  |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: 0 DATES: CAUSES: |  |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: 0 DATES: CAUSES: |  |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |  |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: 10/1998 CAPTURE EFFICIENCY: 100% DESTRUCTION EFFICIENCY: 97.7% OVERALL EFFICEINCY: 97.7% SOURCE TEST/PERFORMANCE DATA: Inlet: 7,530 dscfm, 1729 ppmv (24.73 lbs/hr) VOC measured as C (w/o CH4). Outlet: 7,960 dscfm, 37 ppmv (0.559 lbs/hr) VOC measured as C (w/o CH4) OPERATING CONDITIONS: TEST METHODS: SCAQMD Test Method 25.1, EPA Method 204 |  |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: 321072 |
| --- | --- | --- |
| The facility has a total of eleven flexographic printing presses , seven of these presses are vented to air pollution control (APC), and four are not. In the four printing presses that are not vented to APC, the facility uses graphic arts materials that are in compliance with Rule 1130(c)(1) and BACT limit. In the seven printing presses that are vented to APC, the facility uses graphic arts materials with VOC contents higher than Rule 1130(c)(1) limits, but the VOC emissions are captured within four permanent total enclosures and all are vented to APC with 97.7% overall tested control efficiency, exceeding the required overall control efficiency of 75% for Rule 1130(c)(4), 86% for Rule 1171(c)(5), and 95% for BACT. (In order to use solvent based inks and alcohols as cleanup materials, Poly Pak proposed to install permanent total enclosure and regenerative thermal oxidizer. Permit to construct was issued to this control device in 1997. The control device was tested in 1998 achieving 97.7% overall control efficiency. The operation of this control device since 1998 at this high overall control efficiency sets the current standard for BACT. Permit to operate for this control device was isssued in April 2000 requiring a 95% overall control efficiency. The 95% overall control efficiency reflects current BACT standard for flexographic printing presses using solvent based inks.) |  |  |