Application No.:  389926

Equipment Category – Heater - Other Process

| 1. GENERAL INFORMATION |  |  | DATE: 7/17/2002 |
| --- | --- | --- | --- |
| a. manufacturer: Selas Fluid Processing, Corp. |  |  |  |
| b. type: Hydrogen Reformer |  | c. model: LH 1207 B001 |  |
| d. style: |  |  |  |
| e. applicable aqmd regulation xi rules: 1146 |  |  |  |
| f. cost: $NA () source of cost data: |  |  |  |
| g. operating schedule: 24 hrs/day 7 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 389926 |
| --- | --- | --- | --- |
| A. FUNCTION: Provides heat to catalyst-filled tubes in which steam-natural gas mixture is reformed to hydrogen-rich gas. |  |  |  |
| B. maximum Heat Input: 117.6 MMBtu/hr |  | C. maximum throughput: |  |
| D. burner information: no.: 18 type: Zeeco low-NOx (fuel-staged) |  |  |  |
| e. primary fuel: Reformer Waste Gas (PSA tail gas) |  | f. other fuel: nATURAL gAS (wARMUP AND SUPPLEMENT) |  |
| g. operating conditions: Steady, full-load |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 389926 |  |
| --- | --- | --- | --- | --- |
| A. NAME: Praxair |  |  |  | B. SIC CODE: 2813 |
| C. ADDRESS: 5705 East Airport Drive CITY: Ontario STATE: CA ZIP: 91761 |  |  |  |  |
| D. CONTACT PERSON: Wayne Salmi |  | E. PHONE NO.: 909-974-5137 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: 389926 |
| --- | --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Arman Villena |  |  | D. PHONE NO.: 909-396-3356 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: 389926 (A/N) ISSUANCE DATE: 3/15/2002 check if no p/c P/O NO.: ISSUANCE DATE: |  |  |  |  |
| F. START-UP DATE: |  |  |  |  |

| 5. EMISSION INFORMATION |  |  | App. No.: 389926 |  |  |
| --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |
| A1. PERMIT LIMIT: ppmvd@3%O2 (1-hr Avg.): NOx-5, CO-400, NH3-5 |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: Concentration limits in flue gas from process heater of 5 ppm NOx and 5 ppm NH3 (ppmvd@3%O2, 1-hr Avg.) |  |  |  |  |  |
| a3. BASIS OF THE BACT/LAER DETERMINATION: Current LAER standard for process heater with SCR |  |  |  |  |  |
| ##### B. CONTROL TECHNOLOGY |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Peerless |  |  |  |  |  |
| B2. TYPE: SCR |  |  |  |  |  |
| B3. DESCRIPTION: Ammonia is mixed into flue gas, and the mixture is passed through a catalyst bed, in which the ammonia reacts with NOx reducing it to N2. SCR design operating temperature is 760F, and design removal efficiency is 90%. |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: 389927 (A/N) ISSUANCE DATE: 3/15/2002 P/O NO.: ISSUANCE DATE: |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: 92,700 lb/hr (approx. 20,000 scfm) ACTUAL CONTAMINANT LOADING: 50 ppmvd@3%O2 (expected) BLOWER HP: |  |  |  |  |  |
| B6. WARRANTY: |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: NOx, CO, ROG, PM, SOx |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: NH3 |  |  |  |  |  |
| B9. SPACE REQUIREMENT: Reactor dimensions: 5.2'Wx17.8'Lx5.5'D |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  | ###### B11. UNUSED |  |
| B12. OPERATING HISTORY: Under Construction or Starting Up |  |  |  |  |  |
| ### B13. UNUSED |  | ### B14. UNUSED |  |  |  |
| ##### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in EQUIPMENT cost EQUIPMENT: $NA INSTALLATION: $NA () SOURCE OF COST DATA: |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $NA () SOURCE OF COST DATA: |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: DATES: CAUSES: |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: DATES: CAUSES: |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: CAPTURE EFFICIENCY: DESTRUCTION EFFICIENCY: OVERALL EFFICEINCY: SOURCE TEST/PERFORMANCE DATA: OPERATING CONDITIONS: TEST METHODS: |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: 389926 |
| --- | --- | --- |
| A source test is required within 90 days after startup for NOx, CO, ROG, NH3, PM, SOx. The facility is in RECLAIM, and will have CEMS for NOx and O2. The permit requires CEMS for NH3 also; however, the applicant is requesting to substitute a parametric monitoring method. |  |  |