Application No.:	341340

Equipment Category – Heater - Refinery

| 1. GENERAL INFORMATION |  |  | DATE: 7/14/1999 |
| --- | --- | --- | --- |
| a. manufacturer: Born Heaters |  |  |  |
| b. type: |  | c. model: |  |
| d. style: |  |  |  |
| e. applicable aqmd regulation xi rules: None (This facility is a RECLAIM facility. All previously applicable Regulation XI rules have been supplanted by RECLAIM rules - See AQMD Regulation XX - RECLAIM) |  |  |  |
| f. cost: $12,000,000 (1999) source of cost data: |  |  |  |
| g. operating schedule: 24 hrs/day 7 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 341340 |
| --- | --- | --- | --- |
| A. FUNCTION: This reformer furnace is located at the Steam Naphtha Reformer plant, and is used for hydrogen production. |  |  |  |
| B. maximum Heat Input: 653 MMBtu/Hr |  | C. maximum throughput: 85 Million SCF of H2 per day |  |
| D. burner information: no.: 384 type: Low NOX |  |  |  |
| e. primary fuel: Refinery Gas |  | f. other fuel: Natural Gas |  |
| g. operating conditions: Variable Load |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 341340 |
| --- | --- | --- | --- |
| A. NAME: Chevron Products CO. |  |  |  |
| B. ADDRESS: 324 W. El Segundo Blvd., CITY: El Segundo STATE: CA ZIP: 90245 |  |  |  |
| C. CONTACT PERSON: Charlie Aarni |  | D. PHONE NO.: (310) 615-5285 |  |

| 4. PERMIT INFORMATION |  |  | App. No.: 341340 |
| --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  |  |  |
| B. AGENCY CONTACT PERSON: Pablo L. Pua |  | C. PHONE NO.: (909) 396-2597 |  |
| D. PERMIT TO CONSTRUCT INFORMATION: P/C NO.: 341340 ISSUANCE DATE: 3/24/1999 |  |  |  |
| E. START-UP DATE: 2/28/2000 |  |  |  |
| F. PERMIT TO OPERATE INFORMATION: P/O NO.: N/A ISSUANCE DATE: |  |  |  |

| 5. EMISSION INFORMATION |  |  | App. No.: 341340 |
| --- | --- | --- | --- |
| A. PERMIT |  |  |  |
| A1. PERMIT LIMIT: NOX: 5 ppmvd corrected to 3% O2, 3-hour average CO: 25 ppmvd corrected to 3% O2, 3-hour average NH3: 20 ppmvd corrected to 3% O2, 3-hour average |  |  |  |
| A2. bact/lAer DETERMINATION: The BACT/LAER for NOx, CO, and NH3 emissions from this heater are shown in Item (5a1). These determinations are based on: 1) an AQMD Permit to Construct (A/N 337979) with 5 ppm NOx limit issued for a reformer heater with SCR to Air Products and Chemical Inc.; 2) an AQMD Permit to Construct (A/N 343185) with a 5 ppm NOx limit issued for an industrial boiler with a Cannon Technologies low temperature oxidation (LTO) system to Fansteel/California Drop Forge; and 3) the Altadena Dairy boiler (A/N 259724) with Cannon LTO which has been demonstrated to meet 5 ppm NOX at 3% O2, dry. However, AQMD has now determined that the LTO System does not meet the achieved-in-practice criteria of 6 months of continuous operation at 50% or more capacity. Therefore, the NOx emission limit may not be considered achieved in practice BACT at this time. |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Haldor Topsoe |  |  |  |
| B2. TYPE: Selective Catalytic Reduction System |  |  |  |
| B3. DESCRIPTION: SCR system has been used to chemically reduces NOX emissions from combustion sources such as heaters, boilers, and gas turbines. This chemical reation involves: (1) injecting anhydrous ammonia (NH3), aqueous ammonia, or aqueous urea into the flue gas; (2) mixing NH3 with NOX in the combustion gases upstream of a NOX reactor; and (3) a series of chemical reactions of the mixture in the presence of oxygen and a catalyst bed (enclosed in the NOX reactor) to form N2 and water. This SCR system is most effective whenever the reaction occurs within a temperature envelop of 500 and 750 degree F. |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: 341341 ISSUANCE DATE: 3/24/1999 P/O NO.: N/A ISSUANCE DATE: |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: N/A ACTUAL CONTAMINANT LOADING: N/A BLOWER HP: N/A HP |  |  |  |
| B6. WARRANTY: Unknown |  |  |  |
| B7. PRIMARY POLLUTANTS: The primary pollutants are VOC, NOX, CO, SOX, and PM10. |  |  |  |
| B8. SECONDARY POLLUTANTS: NH3 |  |  |  |
| B9. SPACE REQUIREMENT: 94 sq. ft Platform |  |  |  |
| B10. LIMITATIONS: |  |  |  |
| B11. LOCATION OF PRIOR DEMONSTRATION & AGENCY: FACILITY: Tosco Refining CO./Altadena Diary CONTACT PERSON: David Tobler/Ed Goren PHONE NO.: (310) 952-6297/ (626)854-4202 AGENCY: SCAQMD ADDRESS: CONTACT PERSON: Ngoc Tran/Knut Berulsen PHONE NO.: (909) 396-2602/3136 |  |  |  |
| B12. OPERATING HISTORY: Heater/SCR has not been installed |  |  |  |
| B13. SOURCE TEST/PERFORMANCE DATA ANALYSIS: DATE OF SOURCE TEST: CAPTURE EFFICIENCY: DESTRUCTION EFFICIENCY: OVERALL EFFICEINCY: PERFORMANCE DATA: |  |  |  |
| B14. SOURCE TEST CONDITIONS/PERFORMANCE DATA: Heater/SCR has not been installed |  |  |  |
| C. COST |  |  |  |
| C1. CONTROL EQUIPMENT COST: check if installation cost is included in capital cost CAPITAL: $400,000 INSTALLATION: $Unknown ( ) SOURCE OF COST DATA: |  |  |  |
| C2. ANNUAL OPERATIONAL/MAINTENANCE COST: $70,000 (1999) SOURCE OF COST DATA: |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: Heater/SCR has not been installed |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: 0 DATES: CAUSES: N/A |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: 0 DATES: CAUSES: N/A |  |  |  |
| D5. FREQUENCY OF MAINTENANCE: Heater/SCR has not been installed |  |  |  |

| 6. COMMENTS |  | App. No.: 341340 |
| --- | --- | --- |