Application No.:  359876

Equipment Category – I.C. Engine - Emergency, Spark Ignition

| 1. GENERAL INFORMATION |  |  | DATE: 10/2/1999 |
| --- | --- | --- | --- |
| a. manufacturer: Waukesha |  |  |  |
| b. type: spark ignition |  | c. model: L5790G |  |
| d. style: |  |  |  |
| e. applicable aqmd regulation xi rules: exempt (flood control) |  |  |  |
| f. cost: $ ( ) source of cost data: |  |  |  |
| g. operating schedule: 1 hrs/day 1 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 359876 |
| --- | --- | --- | --- |
| A. FUNCTION: The engine drives an emergency flood control pump. |  |  |  |
| B. maximum Heat Input: |  | C. maximum throughput: 750 BHP |  |
| D. burner information: no.: type: |  |  |  |
| e. primary fuel: natural gas |  | f. other fuel: lpg standby fuel |  |
| g. operating conditions: |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 359876 |
| --- | --- | --- | --- |
| A. NAME: Orange County Flood Control District |  |  |  |
| B. ADDRESS: 300 North Flower St CITY: Santa Ana STATE: CA ZIP: 92702 |  |  |  |
| C. CONTACT PERSON: Vince Rosales |  | D. PHONE NO.: 714-834-2316 |  |

| 4. PERMIT INFORMATION |  |  | App. No.: 359876 |
| --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  |  |  |
| B. AGENCY CONTACT PERSON: Roy Olivares |  | C. PHONE NO.: 909-393-2208 |  |
| D. PERMIT TO CONSTRUCT INFORMATION: P/C NO.: 359876 ISSUANCE DATE: |  |  |  |
| E. START-UP DATE: P/C to P/O |  |  |  |
| F. PERMIT TO OPERATE INFORMATION: P/O NO.: F22341 ISSUANCE DATE: 10/5/1999 |  |  |  |

| 5. EMISSION INFORMATION |  |  | App. No.: 359876 |
| --- | --- | --- | --- |
| A. PERMIT |  |  |  |
| A1. PERMIT LIMIT: Engine operation less than 200 hrs/yr (recordkeeping required), and the following emission limits (grams/bhp-hr): NOx = 0.15, VOC = 0.15, and CO = 0.6 |  |  |  |
| A2. bact/lAer DETERMINATION: (grams/bhp-hr) VOC = 0.15, NOx = 0.15, CO = 0.6 (AQMD is reconsidering the BACT requirement for future applications of this type.) |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Miratech |  |  |  |
| B2. TYPE: 3-way catalytic converter and air/fuel ratio controller |  |  |  |
| B3. DESCRIPTION: The NOx, VOC, and CO emissions in the exhaust gas are converted to N2, CO2/H2O, and CO2, respectively, in the presence of a platinum/rhodium based catalyst. The air/fuel ratio controller ensures that the O2 concentration in the exhaust gas stays at or below 0.5% (the optimum exhaust gas mixture favored by the catalyst). |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: none ISSUANCE DATE: P/O NO.: ISSUANCE DATE: |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: 3828 acfm ACTUAL CONTAMINANT LOADING: BLOWER HP: |  |  |  |
| B6. WARRANTY: |  |  |  |
| B7. PRIMARY POLLUTANTS: NOx, VOC, and CO |  |  |  |
| B8. SECONDARY POLLUTANTS: none |  |  |  |
| B9. SPACE REQUIREMENT: |  |  |  |
| B10. LIMITATIONS: |  |  |  |
| B11. LOCATION OF PRIOR DEMONSTRATION & AGENCY: FACILITY: CONTACT PERSON: PHONE NO.: AGENCY: ADDRESS: CONTACT PERSON: PHONE NO.: |  |  |  |
| B12. OPERATING HISTORY: |  |  |  |
| B13. SOURCE TEST/PERFORMANCE DATA ANALYSIS: DATE OF SOURCE TEST: CAPTURE EFFICIENCY: DESTRUCTION EFFICIENCY: OVERALL EFFICEINCY: PERFORMANCE DATA: |  |  |  |
| B14. SOURCE TEST CONDITIONS/PERFORMANCE DATA: |  |  |  |
| C. COST |  |  |  |
| C1. CONTROL EQUIPMENT COST: check if installation cost is included in capital cost CAPITAL: $ INSTALLATION: $ ( ) SOURCE OF COST DATA: |  |  |  |
| C2. ANNUAL OPERATIONAL/MAINTENANCE COST: $ ( ) SOURCE OF COST DATA: |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: DATES: CAUSES: |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: DATES: CAUSES: |  |  |  |
| D5. FREQUENCY OF MAINTENANCE: |  |  |  |

| 6. COMMENTS |  | App. No.: 359876 |
| --- | --- | --- |
| Engine is limited to 200 hrs/yr, catalytic converter sized for emissions limits specified in section 5A2, and a source test is not required. |  |  |