Application No.:  359091

Equipment Category – I.C. Engine - Fire Pump, Spark Ignition

| 1. GENERAL INFORMATION |  |  | DATE: 10/20/1999 |
| --- | --- | --- | --- |
| a. manufacturer: Ford |  |  |  |
| b. type: spark ignition |  | c. model: LSG-875R-6003-C |  |
| d. style: |  |  |  |
| e. applicable aqmd regulation xi rules: exempt (emergency standby) |  |  |  |
| f. cost: $ (1999) source of cost data: |  |  |  |
| g. operating schedule: 1 hrs/day 1 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 359091 |
| --- | --- | --- | --- |
| A. FUNCTION: Engine drives an emergency fire pump |  |  |  |
| B. maximum Heat Input: |  | C. maximum throughput: 193 BHP |  |
| D. burner information: no.: type: |  |  |  |
| e. primary fuel: Gasoline |  | f. other fuel: |  |
| g. operating conditions: |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 359091 |
| --- | --- | --- | --- |
| A. NAME: Disneyland Resort |  |  |  |
| B. ADDRESS: 1313 Harbor Blvd. CITY: Anaheim STATE: CA ZIP: 92803 |  |  |  |
| C. CONTACT PERSON: Tom Marsden |  | D. PHONE NO.: 714-781-4501 |  |

| 4. PERMIT INFORMATION |  |  | App. No.: 359091 |
| --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  |  |  |
| B. AGENCY CONTACT PERSON: Roy Olivares |  | C. PHONE NO.: 909-396-2208 |  |
| D. PERMIT TO CONSTRUCT INFORMATION: P/C NO.: p/o no p/c ISSUANCE DATE: 10/21/1999 |  |  |  |
| E. START-UP DATE: Class III |  |  |  |
| F. PERMIT TO OPERATE INFORMATION: P/O NO.: F22234 ISSUANCE DATE: 10/21/1999 |  |  |  |

| 5. EMISSION INFORMATION |  |  | App. No.: 359091 |
| --- | --- | --- | --- |
| A. PERMIT |  |  |  |
| A1. PERMIT LIMIT: 200 hrs/yr |  |  |  |
| A2. bact/lAer DETERMINATION: (gr/bhp-hr) VOC: 0.15, NOx: 0.15, CO: 0.6 |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Carsound Exhaust Systems, Inc. |  |  |  |
| B2. TYPE: 3-way catalytic converter and air/fuel ratio controller |  |  |  |
| B3. DESCRIPTION: two banks of 3-way catalytic converters, Carsound, Model 298035, and one Carsound air/fuel ratio controller, Model No. KAT2000 |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: ISSUANCE DATE: P/O NO.: ISSUANCE DATE: |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: ACTUAL CONTAMINANT LOADING: BLOWER HP: HP |  |  |  |
| B6. WARRANTY: |  |  |  |
| B7. PRIMARY POLLUTANTS: NOx, ROG, CO, SOx, PM10 |  |  |  |
| B8. SECONDARY POLLUTANTS: none |  |  |  |
| B9. SPACE REQUIREMENT: |  |  |  |
| B10. LIMITATIONS: |  |  |  |
| B11. LOCATION OF PRIOR DEMONSTRATION & AGENCY: FACILITY: CONTACT PERSON: PHONE NO.: AGENCY: ADDRESS: CONTACT PERSON: PHONE NO.: |  |  |  |
| B12. OPERATING HISTORY: |  |  |  |
| B13. SOURCE TEST/PERFORMANCE DATA ANALYSIS: DATE OF SOURCE TEST: CAPTURE EFFICIENCY: DESTRUCTION EFFICIENCY: OVERALL EFFICEINCY: PERFORMANCE DATA: |  |  |  |
| B14. SOURCE TEST CONDITIONS/PERFORMANCE DATA: |  |  |  |
| C. COST |  |  |  |
| C1. CONTROL EQUIPMENT COST: check if installation cost is included in capital cost CAPITAL: $755.82 INSTALLATION: $ (1999) SOURCE OF COST DATA: |  |  |  |
| C2. ANNUAL OPERATIONAL/MAINTENANCE COST: $ (1999) SOURCE OF COST DATA: |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: DATES: CAUSES: |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: DATES: CAUSES: |  |  |  |
| D5. FREQUENCY OF MAINTENANCE: |  |  |  |

| 6. COMMENTS |  | App. No.: 359091 |
| --- | --- | --- |
| The emissions guarantee provided by the catalyst manufacturer is as follows: (gr/bhp-hr) NO < 0.15, VOC < 0.15, and CO < 0.60 |  |  |