Application No.:	  405533

Equipment Category - Lithographic or Offset Printing, Heatset

| 1. general information |  |  | DATE: 2/26/2003 |
| --- | --- | --- | --- |
| a. manufacturer: Man-Roland |  |  |  |
| b. type: Web-Fed |  | c. model: Rotoman 8 Color |  |
| d. style: Has automatic blanket washing system |  |  |  |
| e. applicable aqmd rules: 407, 409, 1130, 1171 |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: 16 hrs/day 6 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 405533 |
| --- | --- | --- | --- |
| A. FUNCTION: Prints brochures, advertising and posters. Dryer is MegTec Systems Model Dual-Dry III-135--gas fired, 5.4 MMBtu/hr maximum input, Maxon Cyclomax EB4MR low-NOx burner, three circulating fans (40, 30 and 10 hp), and 7.5 hp exhaust fan. |  |  |  |
| b. size/dimension/capacity: 38" web width |  |  |  |
| c. blowers: 7.5 hp exhaust fan |  | d. total flow rate: scfm |  |
| e. material stored/processed/handled: |  |  |  |
| f. throughput/process rate/usage rate: |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 405533 |  |
| --- | --- | --- | --- | --- |
| A. NAME: Madison-Graham Colorgraphics, Inc. |  |  |  | B. SIC CODE: 2752 |
| C. ADDRESS: 150 North Myers Street CITY: Los Angeles STATE: CA ZIP: 90033 |  |  |  |  |
| D. CONTACT PERSON: Pamela Jones |  | E. PHONE NO.: 323-261-7171 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: 405533 |
| --- | --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Farah Milner |  |  | D. PHONE NO.: 909-396-2518 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: 405533 ISSUANCE DATE: 12/18/2002 check if no p/c P/O NO.: ISSUANCE DATE: |  |  |  |  |
| F. START-UP DATE: Press is being installed now (January 2003) |  |  |  |  |

| 5. EMISSION INFORMATION |  |  |  | App. No.: 405533 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |  |
| A1. PERMIT LIMIT: Compliance with Rules 1130 and 1171. Total VOC emissions form facility not to exceed 10,890 lb in any calendar month. Composite vapor pressure of blanket and roller washes not to exceed 5 mm Hg at 20 C. VOC content of fountain solution, as applied, including water and exempt solvents, not to exceed 0.10 lb/gal. Dryer must be vented to control equipment--regenerative thermal oxidizer (RTO). RTO conditions (A/N 403986): minimum chamber temperature 1400F, minimum overall control (collection and destruction) efficiency 95%. |  |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: VOC in fountain solution <8% (vol.), vapor pressures of blanket and roller washes <10 mm Hg at 20C, NOx from ovens not to exceed 30 ppm (dry, 3% O2), afterburner with minimum chamber temperature 1400F, minimum residence time 0.3 sec and minimum overall control efficiency of 95%. |  |  |  |  |  |  |
| A3. BASIS OF THE BACT DETERMINATION: Part D of AQMD BACT Guidelines. |  |  |  |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: MegTec |  |  |  |  |  |  |
| B2. TYPE: Regenerative Thermal Oxidizer, Model Millenium 1129-95, Maxon Kinemax 3 Series G burner, 2.5 MMBtu/hr maximum input. |  |  |  |  |  |  |
| B3. DESCRIPTION: VOC-laden air from three web-fed lithographic heatset presses enters through bed of hot ceramic material where it is preheated and passes to a combustion chamber for burnout of VOC. The hot combustion gas exits through a second bed of ceramic material, which it heats. When the inlet bed is cooled below some minimum temperature, the flow path is reversed by valve actions so that the inlet bed becomes the outlet bed and visa versa. |  |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: 403986 ISSUANCE DATE: 12/18/2002 P/O NO.: ISSUANCE DATE: |  |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: 10,800 scfm ACTUAL CONTAMINANT LOADING: Approx. 74 lb/hr BLOWER HP: 125 hp |  |  |  |  |  |  |
| B6. WARRANTY: One year parts and labor. There is also a performance guarantee of 99% destruction efficiency or 20 ppmv as methane, based on use of U.S. EPA Method 25A. |  |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: VOC, NOx, CO, PM |  |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: NOx, CO |  |  |  |  |  |  |
| B9. SPACE REQUIREMENT: |  |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  |  |  | B11. UNUSED |
| B12. OPERATING HISTORY: |  |  |  |  |  |  |
| B13. UNUSED |  |  | B14. UNUSED |  |  |  |
| ### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in capital cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: |  |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: DATES: CAUSES: |  |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: DATES: CAUSES: |  |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |  |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: CAPTURE EFFICIENCY: DESTRUCTION EFFICIENCY: OVERALL EFFICIENCY: SOURCE TEST/PERFORMANCE DATA: OPERATING CONDITIONS: TEST METHODS: |  |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: 405533 |
| --- | --- | --- |
| The fountain solution is made up from Allied Press Control concentrate (1.7 lb/gal VOC) diluted with 128 gal water per 3 gal concentrate. The washes are either Autowash 6000 One-Step Blanket & Roller Wash (2.7 mm Hg at 20 C) or Roller Wash A230 and Envirowash 220 Automatic Blanket Wash (composite vapor pressure of latter two products is 2.8 mm Hg at 20 C). These products have all been in use at this facility on other web-fed, heatset lithographic presses for more than six months. |  |  |