Application No.:  385874

Equipment Category – Dryer or Oven

| 1. GENERAL INFORMATION |  |  | DATE: 3/12/2003 |
| --- | --- | --- | --- |
| a. manufacturer: Ferry Industries |  |  |  |
| b. type: |  | c. model: RS4-280 |  |
| d. style: |  |  |  |
| e. applicable aqmd rules: 2012 |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: 24 hrs/day 6 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 385874 |
| --- | --- | --- | --- |
| A. FUNCTION: Polyethylene resin melting and curing. Plastic toy manufacturing (molding) line. This oven (D22) is one of six curing ovens (D20-D25) in this facility. |  |  |  |
| B. maximum Heat Input: 5.4 MMBtu/hr |  | C. maximum throughput: 4,600 lb/hr |  |
| D. burner information: no.: 1 type: Low-NOx |  |  |  |
| e. primary fuel: Natural Gas |  | f. other fuel: None |  |
| g. operating conditions: Steady |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 385874 |  |
| --- | --- | --- | --- | --- |
| A. NAME: Newellrubbermaid |  |  |  | B. SIC CODE: 3089 |
| C. ADDRESS: 19605 East Walnut Drive N. CITY: City of Industry STATE: CA ZIP: 91789 |  |  |  |  |
| D. CONTACT PERSON: Luis Lopez |  | E. PHONE NO.: 909-598-4007 x240 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: 385874 |
| --- | --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Doug Gordon |  |  | D. PHONE NO.: 909-396-2683 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: ISSUANCE DATE: check if no p/c P/O NO.: 385874 ISSUANCE DATE: 12/7/2001 |  |  |  |  |
| F. START-UP DATE: 4/12/2001 |  |  |  |  |

| 5. EMISSION INFORMATION |  |  | App. No.: 385874 |  |  |
| --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |
| A1. PERMIT LIMIT: NOx not to exceed 18 ppmvd@3%O2 (RECLAIM Concentration Limit) |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: 18 ppmvd@3%O2 (low-NOx burner) |  |  |  |  |  |
| a3. BASIS OF THE BACT/LAER DETERMINATION: Achieved in Practice |  |  |  |  |  |
| ##### B. CONTROL TECHNOLOGY |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Maxon |  |  |  |  |  |
| B2. TYPE: Low-NOx Burner |  |  |  |  |  |
| B3. DESCRIPTION: Cyclomax |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: ISSUANCE DATE: P/O NO.: ISSUANCE DATE: |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: ACTUAL CONTAMINANT LOADING: BLOWER HP: |  |  |  |  |  |
| B6. WARRANTY: NOx guaranteed not to exceed 15 ppmvd@3%O2, Maxon brochure states that CO will not exceed 75 ppmvd@3%O2 |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: NOx, CO, VOC, PM |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: |  |  |  |  |  |
| B9. SPACE REQUIREMENT: |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  | ###### B11. UNUSED |  |
| B12. OPERATING HISTORY: The burner has been in regular service and has performed well since startup. |  |  |  |  |  |
| ### B13. UNUSED |  | ### B14. UNUSED |  |  |  |
| ##### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in EQUIPMENT cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: None DATES: CAUSES: |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: None DATES: CAUSES: |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: 4/12/2001 CAPTURE EFFICIENCY: DESTRUCTION EFFICIENCY: OVERALL EFFICIENCY: SOURCE TEST/PERFORMANCE DATA: Raw NOx (1-hr avg.), ppmvd 2.26 O2, % (dry) 16.06 Correct NOx (1-hr avg.), ppmvd@3%O2 8.4 OPERATING CONDITIONS: TEST METHODS: This test was performed after installation of the Cyclomax burner on D22 only. Based on the results of this test, the cyclomax burner was installed in the other five ovens. This test was approved by the permitting team, and the Permit to Operate was issued on this basis subject to the Rule 2012 condition requiring source testing of all six ovens within 12 months.. |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: 385874 |
| --- | --- | --- |
| This facility installed these burners and requested the 18 ppm limits on the ovens as RECLAIM concentration limits in order to meet its RECLAIM NOx allocations. A source test was required within 12 months after the Permit to Operate was issued. However, the applicant believed that he was not required to submit the source test to AQMD but only to keep it on file. |  |  |