Application No.:  347376

Equipment Category – Dryer or Oven - Others, Direct and Indirect Fired

| 1. GENERAL INFORMATION |  |  | DATE: 9/22/1999 |
| --- | --- | --- | --- |
| a. manufacturer: |  |  |  |
| b. type: CONTINUOUS, DIRECT-FIRED |  | c. model: |  |
| d. style: |  |  |  |
| e. applicable aqmd regulation xi rules: None |  |  |  |
| f. cost: $ ( ) source of cost data: |  |  |  |
| g. operating schedule: 24 hrs/day 7 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 347376 |
| --- | --- | --- | --- |
| A. FUNCTION: TO SOFTEN POLYSTYRENE SHEET |  |  |  |
| B. maximum Heat Input: 4,000,000 BTU/HR |  | C. maximum throughput: |  |
| D. burner information: no.: 4 type: MAXON, MODEL 408 |  |  |  |
| e. primary fuel: N. G. |  | f. other fuel: |  |
| g. operating conditions: 550 DEGREES FAHRENHEIT |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 347376 |
| --- | --- | --- | --- |
| A. NAME: DART CONTAINER CORPORATION OF CALIFORNIA |  |  |  |
| B. ADDRESS: 150 S MAPLE ST CITY: CORONA STATE: CA ZIP: 91720 |  |  |  |
| C. CONTACT PERSON: KIM KEENER |  | D. PHONE NO.: (909) 7358115 |  |

| 4. PERMIT INFORMATION |  |  | App. No.: 347376 |
| --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  |  |  |
| B. AGENCY CONTACT PERSON: MARK LIU |  | C. PHONE NO.: (909) 396-2538 |  |
| D. PERMIT TO CONSTRUCT INFORMATION: P/C NO.: P/C ISSUANCE DATE: 9/10/1999 |  |  |  |
| E. START-UP DATE: DECEMBER 1999 |  |  |  |
| F. PERMIT TO OPERATE INFORMATION: P/O NO.: NA ISSUANCE DATE: |  |  |  |

| 5. EMISSION INFORMATION |  |  | App. No.: 347376 |
| --- | --- | --- | --- |
| A. PERMIT |  |  |  |
| A1. PERMIT LIMIT: (1) THE TEMPERATURE IN THE OVEN SHALL NOT EXCEED 550 DEGREES FAHRENHEIT, (2) NO BLOWING AGENT SHALL BE PRESENT IN THE POLYSTYRENE SHEET PROCESSED IN THE OVEN, (3) THE NOX EMISSIONS IN THE OVEN EXHAUST SHALL NOT EXCEED 30 PPM CORRECTED TO 3% OXYGEN, (4) THE CO EMISSIONS IN THE OVEN EXHAUST SHALL NOT EXCEED 2000 PPM CORRECTED TO 3% OXYGEN, AND (5) THE PM EMISSIONS IN THE OVEN EXHAUST SHALL NOT EXCEED 0.1 GRAINS/SCF. |  |  |  |
| A2. bact/lAer DETERMINATION: NOX EMISSIONS =< 30 PPM AT 3% OXYGEN |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |
| B1. MANUFACTURER/SUPPLIER: MAXON |  |  |  |
| B2. TYPE: |  |  |  |
| B3. DESCRIPTION: MODEL 408 |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: SAME AS OVEN ISSUANCE DATE: 9/10/1999 P/O NO.: ISSUANCE DATE: |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: ACTUAL CONTAMINANT LOADING: BLOWER HP: HP |  |  |  |
| B6. WARRANTY: |  |  |  |
| B7. PRIMARY POLLUTANTS: NOX, CO, VOC, SOX, AND PM10 |  |  |  |
| B8. SECONDARY POLLUTANTS: NONE |  |  |  |
| B9. SPACE REQUIREMENT: |  |  |  |
| B10. LIMITATIONS: |  |  |  |
| B11. LOCATION OF PRIOR DEMONSTRATION & AGENCY: FACILITY: UNKNOWN CONTACT PERSON: PHONE NO.: AGENCY: ADDRESS: CONTACT PERSON: PHONE NO.: |  |  |  |
| B12. OPERATING HISTORY: ONE OF THE OVENS WILL BE INSTALLED IN DECEMBER 1999 |  |  |  |
| B13. SOURCE TEST/PERFORMANCE DATA ANALYSIS: DATE OF SOURCE TEST: THEY WILL BE TESTED CAPTURE EFFICIENCY: DESTRUCTION EFFICIENCY: OVERALL EFFICEINCY: PERFORMANCE DATA: |  |  |  |
| B14. SOURCE TEST CONDITIONS/PERFORMANCE DATA: |  |  |  |
| C. COST |  |  |  |
| C1. CONTROL EQUIPMENT COST: check if installation cost is included in capital cost CAPITAL: $UNKNOWN INSTALLATION: $UNKNOWN ( ) SOURCE OF COST DATA: |  |  |  |
| C2. ANNUAL OPERATIONAL/MAINTENANCE COST: $UNKNOWN ( ) SOURCE OF COST DATA: |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: DATES: CAUSES: |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: DATES: CAUSES: |  |  |  |
| D5. FREQUENCY OF MAINTENANCE: |  |  |  |

| 6. COMMENTS |  | App. No.: 347376 |
| --- | --- | --- |