Application No.:  385818

Equipment Category – Dryer or Oven

| 1. GENERAL INFORMATION |  |  | DATE: 5/13/2003 |
| --- | --- | --- | --- |
| a. manufacturer: Industrial Process Equipment |  |  |  |
| b. type: Conveyorized |  | c. model: |  |
| d. style: |  |  |  |
| e. applicable aqmd rules: |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: 8 hrs/day 5 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 385818 |
| --- | --- | --- | --- |
| A. FUNCTION: Powder coat curing oven, 400-600F operating temperature. |  |  |  |
| B. maximum Heat Input: 5 MMBtu/hr |  | C. maximum throughput: |  |
| D. burner information: no.: 1 type: Low-NOx |  |  |  |
| e. primary fuel: Natural Gas |  | f. other fuel: None |  |
| g. operating conditions: Eight hours per day. |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 385818 |  |
| --- | --- | --- | --- | --- |
| A. NAME: Fletcher Coating |  |  |  | B. SIC CODE: 3479 |
| C. ADDRESS: 426 Fletcher Ave. CITY: Orange STATE: CA ZIP: 92865 |  |  |  |  |
| D. CONTACT PERSON: Kurtis Breeding |  | E. PHONE NO.: 714-637-4763 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: 385818 |
| --- | --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Fred Del Rosario |  |  | D. PHONE NO.: 909-396-2663 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: F48686 ISSUANCE DATE: 2/6/2002 check if no p/c P/O NO.: F48686 ISSUANCE DATE: 2/6/2002 |  |  |  |  |
| F. START-UP DATE: December 2002 |  |  |  |  |

| 5. EMISSION INFORMATION |  |  | App. No.: 385818 |  |  |
| --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |
| A1. PERMIT LIMIT: 30 ppmvd NOx, corrected to 3% O2, 30-minute average. Facility-wide VOC limit of 780 lb per calendar month. |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: Low-NOx Burner |  |  |  |  |  |
| a3. BASIS OF THE BACT/LAER DETERMINATION: |  |  |  |  |  |
| ##### B. CONTROL TECHNOLOGY |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Eclipse |  |  |  |  |  |
| B2. TYPE: Low-NOx |  |  |  |  |  |
| B3. DESCRIPTION: WINNOX Model WX-500 |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: ISSUANCE DATE: P/O NO.: ISSUANCE DATE: |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: ACTUAL CONTAMINANT LOADING: BLOWER HP: |  |  |  |  |  |
| B6. WARRANTY: Burner manufacturer literature shows NOx <30 and CO <250 (both ppmvd@3%O2) down to 10% of rated input. |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: NOx, CO, VOC PM |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: |  |  |  |  |  |
| B9. SPACE REQUIREMENT: |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  | ###### B11. UNUSED |  |
| B12. OPERATING HISTORY: Oven started operation in December 2002 and has been in service without problems since that time. The oven has been used only about one 8-hr shift per week due to poor market conditions. |  |  |  |  |  |
| ### B13. UNUSED |  | ### B14. UNUSED |  |  |  |
| ##### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in EQUIPMENT cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: None DATES: CAUSES: |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: None related to oven DATES: CAUSES: |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: 12-30-2002 CAPTURE EFFICIENCY: DESTRUCTION EFFICIENCY: OVERALL EFFICIENCY: SOURCE TEST/PERFORMANCE DATA: NOx, ppmvd@3%O2 16 CO, ppmvd@3%O2 14 O2, % (dry) 19.05 Exhaust Flow, dscfm 1560 OPERATING CONDITIONS: Indicated fuel input rate and measured flue gas flow rate indicated that the oven was operating at approximately 20% rated input. Oven temperature was 585F. TEST METHODS: Source test was accepted by AQMD Monitoring & Source Test Engineering group. However, it was noted that the NOx measurement was less than 20% of analyzer range so NOx was only proven to be <20 ppmvd@3%O2. |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: 385818 |
| --- | --- | --- |
| The manufacturer literature indicates that this burner maintains acceptable emissions performance down to approximately 10% of its rated input (10:1 turn-down). This oven requires a maximum turn-down of approximately 5:1. Some ovens require turn-down ratios greater than 10:1, and this burner would not be suitable for those ovens. |  |  |