Application No.:	  364198

Equipment Category – Plastic and Resin Extrusion

| 1. general information |  |  | DATE: 5/1/2002 |
| --- | --- | --- | --- |
| a. manufacturer: Welex |  |  |  |
| b. type: Screw, heavy duty (500 hp) |  | c. model: |  |
| d. style: |  |  |  |
| e. applicable aqmd regulation xi rules: None |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: 16 hrs/day 7 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 364198 |
| --- | --- | --- | --- |
| A. FUNCTION: Extrusion of polystyrene, polyethylene and polypropylene (poly-x) to form sheets. Facility also extrudes acrylonitrile butadiene styrene (ABS) resin to form sheets. |  |  |  |
| b. size/dimension/capacity: Facility includes 10 extruders (5 ABS and 5 poly-x), some have co-extruders. Extruder barrel diameters range from 2.5" to 6". All have scrap grinder/recycle systems. |  |  |  |
| c. blowers: Extruder operates under vacuum |  | d. total flow rate: scfm |  |
| e. material stored/processed/handled: Polystyrene, polyethylene and polypropylene |  |  |  |
| f. throughput/process rate/usage rate: 21.9 MM lb/yr (one extruder line) |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 364198 |  |
| --- | --- | --- | --- | --- |
| A. NAME: Spartech Plastics |  |  |  | B. SIC CODE: 3089 |
| C. ADDRESS: 14263 Gannet Street CITY: La Mirada STATE: CA ZIP: 90638 |  |  |  |  |
| D. CONTACT PERSON: Mark Berger |  | E. PHONE NO.: 714-523-2260 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: 364198 |
| --- | --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Andrew Lee |  |  | D. PHONE NO.: 909-396-2643 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: R-364198 ISSUANCE DATE: 3/24/2000 check if no p/c P/O NO.: F43221 ISSUANCE DATE: 8/18/2001 |  |  |  |  |
| F. START-UP DATE: 1/1/2000 (estimate at time of application) |  |  |  |  |

| 5. EMISSION INFORMATION |  |  |  | App. No.: 364198 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |  |
| A1. PERMIT LIMIT: Minimum afterburner temperature 1400F. All materials added to extruders to be in hardened pellet form, no plasticizers to be added. Cleanup materials not to contain VOC. Facility ABS/acrylic production not to exceed 94,250,000 lb in any 12 month period. |  |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: Extruder vacuum exhaust vented to afterburner with minimum operating temperature of 1400F. Zero-VOC cleanup materials. |  |  |  |  |  |  |
| A3. BASIS OF THE BACT DETERMINATION: Offered by applicant. |  |  |  |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Energy & Environment, Inc. |  |  |  |  |  |  |
| B2. TYPE: Afterburner |  |  |  |  |  |  |
| B3. DESCRIPTION: Direct Flame (Model EE-1000), natural gas fired, 1.6 MMBtu/hr, vents 5 poly-x extruder lines. (Similar afterburner vents 5 ABS extruder lines.) |  |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: A/N 364203 ISSUANCE DATE: 3/16/2000 P/O NO.: F43217 ISSUANCE DATE: 8/18/2001 |  |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: 130 cfm ACTUAL CONTAMINANT LOADING: 57 lb/day VOC (5 extruder lines) BLOWER HP: 5 |  |  |  |  |  |  |
| B6. WARRANTY: |  |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: VOC |  |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: NOx, CO, PM |  |  |  |  |  |  |
| B9. SPACE REQUIREMENT: dimensions are 2' x 18' |  |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  |  |  | B11. UNUSED |
| B12. OPERATING HISTORY: Afterburner has worked well since startup (more than 2 years). Zero-VOC cleanup material is Easy Off Heavy Duty Oven & Grill Cleaner--has been used to clean extruder parts for many years. |  |  |  |  |  |  |
| B13. UNUSED |  |  | B14. UNUSED |  |  |  |
| ### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in capital cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: Jack Nagy DATE: Nov. 2000 |  |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: Operating in compliance |  |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: DATES: CAUSES: |  |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: DATES: CAUSES: |  |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |  |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: CAPTURE EFFICIENCY: DESTRUCTION EFFICIENCY: OVERALL EFFICEINCY: SOURCE TEST/PERFORMANCE DATA: OPERATING CONDITIONS: TEST METHODS: |  |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: 364198 |
| --- | --- | --- |
| The afterburner manufacturer guaranteed 99% destruction efficiency at 1400F chamber temperature. It was estimated that venting the vacuum exhausts of the extruder to the afterburner collects approximately 95% of the VOC released from the process. |  |  |