Application No.:	  352856

Equipment Category – Polyester Resin Operation

| 1. general information |  |  | DATE: 9/23/2003 |
| --- | --- | --- | --- |
| a. manufacturer: |  |  |  |
| b. type: Non-atomizing resin application is used |  | c. model: |  |
| d. style: |  |  |  |
| e. applicable aqmd rules: 1162, 1171 |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: 16 hrs/day 6 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 352856 |
| --- | --- | --- | --- |
| A. FUNCTION: The facility manufactures yachts. In this operation, polyester resins with fiberglass strands or gelcoats are applied to molds. |  |  |  |
| b. size/dimension/capacity: 25'W x 100'L x 20'H |  |  |  |
| c. blowers: One 60 hp |  | d. total flow rate: 12000 scfm |  |
| e. material stored/processed/handled: Polyester resin/fiberglass mixture, gelcoat |  |  |  |
| f. throughput/process rate/usage rate: 400 gal/day resin, 60 gal/day gelcoat |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 352856 |  |
| --- | --- | --- | --- | --- |
| A. NAME: Navigator Yachts |  |  |  | B. SIC CODE: |
| C. ADDRESS: 364 Malbert Street CITY: Perris STATE: CA ZIP: 92370 |  |  |  |  |
| D. CONTACT PERSON: Gil Marshall |  | E. PHONE NO.: 909-657-2117 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: 352856 |
| --- | --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Emmanuel Quizon |  |  | D. PHONE NO.: 909-396-2523 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: 352856 ISSUANCE DATE: 11/24/1999 check if no p/c P/O NO.: F55091 ISSUANCE DATE: 9/24/2002 |  |  |  |  |
| F. START-UP DATE: January 2000 |  |  |  |  |

| 5. EMISSION INFORMATION |  |  |  | App. No.: 352856 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |  |
| A1. PERMIT LIMIT: Pressure drop across filters not to exceed 2" H2O. Compliance with Rule 1162 by use of non-atomized application of resin and gelcoat (required July 1, 2004) and use of compliant materials as follows: maximums of 22% styrene and 14% methyl methacrylate in gelcoat and 35% styrene in resin. Compliance with Rule 1171 (maximum of 25% VOC in cleanup materials). Daily maximum emissions of VOC: facility-61 lb, this system-30 lb. Carbon adsorption system must be regenerated after maximum of 5 days use and must achieve 93.4% overall VOC control. The thermal oxidizer temperature must be maintained at a minimum of 1500F and must achieve a minimum overall VOC control efficiency of 91.4%. |  |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: BACT was determined to be compliance with Rule 1162. Add-on control was elected by the applicant to stay below public notice threshold (Rule 212). Carbon adsorber/thermal oxidizer system achieving 85% overall VOC control is now achieved in practice for this equipment category. |  |  |  |  |  |  |
| A3. BASIS OF THE BACT DETERMINATION: Achieved in practice |  |  |  |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Ship and Shore Environmental |  |  |  |  |  |  |
| B2. TYPE: Carbon adsorption system with on-site regeneration using thermal oxidizer |  |  |  |  |  |  |
| B3. DESCRIPTION: The spray booth is vented to two portable carbon adsorption beds (Model AMK-12J-CB). Each bed is regenerated after every five days use. Regeneration occurs at a different location in the facility by steam stripping the adsorbed VOC into a thermal oxidizer (Model AMJ-1K-50-SGTO). The thermal oxidizer waste heat is recovered in a boiler, which makes steam for the stripping process. |  |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: 352857 ISSUANCE DATE: 11/24/1999 P/O NO.: F55092 ISSUANCE DATE: 9/24/2002 |  |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: 12000 cfm ACTUAL CONTAMINANT LOADING: 2700 ppm as CH4 BLOWER HP: 50 and 7.5 |  |  |  |  |  |  |
| B6. WARRANTY: Carbon adsorber is guaranteed to remove 90% of VOC |  |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: VOC, PM |  |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: NOx, CO, SOx, PM |  |  |  |  |  |  |
| B9. SPACE REQUIREMENT: Each carbon bed is 6' x 9' x 5' deep (4050 lb carbon) |  |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  |  |  | B11. UNUSED |
| B12. OPERATING HISTORY: |  |  |  |  |  |  |
| B13. UNUSED |  |  | B14. UNUSED |  |  |  |
| ### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in capital cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: 5/15/2001-Emmanuel Quizon and Michael Wickson, 9/4/2003-Marty Kay and Howard Lange INSPECTOR'S NAME: DATE: |  |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: 5/15/2001-Inspection and Source Test, 9/4/2003-Inspection |  |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: None DATES: CAUSES: |  |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: None DATES: CAUSES: |  |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |  |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: 2/7/01 and 5/15/2001 CAPTURE EFFICIENCY: 100% DESTRUCTION EFFICIENCY: OVERALL EFFICIENCY: SOURCE TEST/PERFORMANCE DATA: Adsorber Oxidizer Overall Inlet Outlet Inlet Outlet Temperature, F 89.5 98 193 503 Flow Rate, dscfm 5540 7530 192 2450 TGNMO as CH4, ppmv 2704 132 144,855 973 TGNMO, lb/hr 28.41 1.89 52.79 4.52 Removal, % 93.4 91.4 85.37 OPERATING CONDITIONS: Normal TEST METHODS: U.S.EPA Method 204 (Permanent Total Enclosure). AQMD Methods 25.1 and 25.3 (Total Non-Methane Hydrocarbons). Gaseous sampling was performed simultaneously at the inlet and outlet for one hour in each case. The source test was approved by AQMD's Monitoring & Source Test Engineering group. |  |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: 352856 |
| --- | --- | --- |
| Compliance with Rule 1171 is achieved by using acetone as the claenup material. The facility also operates a smaller spray booth (A/N 352855) venting to a similar carbon adsorption system with two portable carbon beds. |  |  |