Application No.:	  402868

Equipment Category – Polyester Resin Operation

| 1. general information |  |  | DATE: 9/23/2003 |
| --- | --- | --- | --- |
| a. manufacturer: Custom |  |  |  |
| b. type: |  | c. model: |  |
| d. style: |  |  |  |
| e. applicable aqmd rules: 1162, 1171 |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: 20 hrs/day 6 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 402868 |
| --- | --- | --- | --- |
| A. FUNCTION: Glassing room, containing nine spray booths and two resin storage tanks. Only non-atomized guns are to be used. Mixing room containing three resin blending tanks. This system is used for manufacture of showers and spas. Resin material contains styrene. Glassing room and mixing room are vented to a VOC control system. The VOC control equipment was relocated from a functionally identical facility at Irvine, CA. The glassing room equipment is new but functionally identical to what was at Irvine. |  |  |  |
| b. size/dimension/capacity: Nine spray booths 14'W x 15'-2"L x 10'-4.5"H each |  |  |  |
| c. blowers: one 5 hp blower per booth |  | d. total flow rate: 57000 scfm |  |
| e. material stored/processed/handled: polyester resin/fiberglass |  |  |  |
| f. throughput/process rate/usage rate: |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 402868 |  |
| --- | --- | --- | --- | --- |
| A. NAME: Jacuzzi Whirlpool Bath |  |  |  | B. SIC CODE: |
| C. ADDRESS: 14880 Monte Vista CITY: Chino STATE: CA ZIP: 91708 |  |  |  |  |
| D. CONTACT PERSON: Susan Faass |  | E. PHONE NO.: 909-548-7732 x324 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: 402868 |
| --- | --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Bijan Ataian |  |  | D. PHONE NO.: 909-396-2454 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: 402868 ISSUANCE DATE: 10/15/2002 check if no p/c P/O NO.: ISSUANCE DATE: |  |  |  |  |
| F. START-UP DATE: Still in "shake down" mode. |  |  |  |  |

| 5. EMISSION INFORMATION |  |  |  | App. No.: 402868 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |  |
| A1. PERMIT LIMIT: Facility-wide VOC limit of 18,030 lb per calendar month. Maximum pressure drops: 0.25" across booth filters and 6" across bag filters. Minimum temperature in cat-ox chamber of 550F. Maximum flow to concentrator or cat-ox - 30,000 scfm. Minimum VOC control efficiencies: concentrator 90%, cat-ox 95%, overall 90%. Glassing room must meet permanent total enclosure criteria in U.S.EPA Method 204. Operator must monitor the cat-ox temperature, flows to the cat-ox and concentrator (check flows weekly), pressure drop across concentrator, and pressure drop across the filter (check weekly). Operator must inspect the filters weekly. Operator must perform annual inspection for visible emissions. Operator must test the oxidation catalyst and zeolite material for activity semi-annually. Equipment must be source tested once in every five year period for VOC emissions and VOC control efficiencies specified in permit. |  |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: Concentrator/catalytic oxidizer system for VOC and filter system for PM10. |  |  |  |  |  |  |
| A3. BASIS OF THE BACT DETERMINATION: |  |  |  |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Engelhard Model RCA37 concentrator, Energy & Environment Model 30000-60C catalytic oxidizer |  |  |  |  |  |  |
| B2. TYPE: Zeolite concentrator and catalytic oxidizer |  |  |  |  |  |  |
| B3. DESCRIPTION: Glassing room is a permanent total enclosure (EPA Method 204) and vented via bag filters to concentrator/cat-ox system. Concentrator contains 12,000 lb zeolite with three zones-adsorption, desorption and cooldown. Total flow from glassing room is 57,000scfm. Of this, 27,000 goes directly to cat-ox, and 30,000 to concentrator. A portion, 3000 scfm, of cat-ox exhaust recirculates through concentrator as desorption gas and back to cat-ox. |  |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: ISSUANCE DATE: P/O NO.: ISSUANCE DATE: |  |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: 57,000 scfm ACTUAL CONTAMINANT LOADING: 250 lb/hr styrene (approx. 1700 ppm as CH4) BLOWER HP: 275 |  |  |  |  |  |  |
| B6. WARRANTY: |  |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: VOC, PM10 |  |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: |  |  |  |  |  |  |
| B9. SPACE REQUIREMENT: Concentrator: 13'-6"W x 3'-6"L x 12'-6"H; Cat-Ox: 11'-8"W x 10'-2"L x 34'-1.5"L. |  |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  |  |  | B11. UNUSED |
| B12. OPERATING HISTORY: The equipment has been operational since December 1, 2002. However, it is still considered to be in a "shake-down" phase. The VOC control system together with functionally identical glassing and mixing equipment had operated previously for approximately 7 months at the Irvine location. |  |  |  |  |  |  |
| B13. UNUSED |  |  | B14. UNUSED |  |  |  |
| ### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in capital cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: Source test required within 120 days after startup |  |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: None DATES: CAUSES: |  |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: None DATES: CAUSES: |  |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |  |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: July 11, 2002 (at Irvine) CAPTURE EFFICIENCY: 100% DESTRUCTION EFFICIENCY: 91.9% OVERALL EFFICIENCY: 91.9% SOURCE TEST/PERFORMANCE DATA: Concentrator removal efficiency: 92.7%, Cat_Ox destruction efficiency: 95.2%, Overall control efficiency 91.9% based on nominal flows to cat-ox and concentrator. OPERATING CONDITIONS: Normal, 80.8 baths per hour production rate. TEST METHODS: AQMD Methods 1.1 (sample point locations), 2.1 (flue gas flow rate), 4.1 (moisture), 25.1 (non-methane organics) and U.S.EPA Method 25A (total hydrocarbons). This test was reviewed and accepted by the permitting team. |  |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: 402868 |
| --- | --- | --- |
| Semi-annual activity check on catalyst will employ a 1.25"-diameter tube furnace. Catalyst sample for test will be obtained by core drilling the face of the catalyst bed to obtain 1.25" x 4" long plug (or 120 cc top/mid/bottom composite sample if spherical catalyst is used). The test will employ nominal 500 ppm (as C1) hexane or toluene in air stream. Inlet and outlet HC will be measured by FID analyzer. The same or similar method can be used for testing the zeolite; however, the applicant has not yet decided. |  |  |