Application No.:	  347406

Equipment Category – Polystyrene Foam Product Manufacturing, Using Blowing Agent

| 1. general information |  |  | DATE: 10/19/99 |
| --- | --- | --- | --- |
| a. manufacturer: GLOUCESTER |  |  |  |
| b. type: DIRECT INJECTION POLYSTYRENE FOAM EXTRUSION |  | c. model: |  |
| d. style: |  |  |  |
| e. applicable aqmd regulation xi rules: 1175 |  |  |  |
| f. cost: $ ( ) source of cost data: |  |  |  |
| g. operating schedule: 24 hrs/day 7 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION | Other equipment |  | App. No.: 347406 |
| --- | --- | --- | --- |
| A. FUNCTION: POLYSTYRENE EXTRUSION |  |  |  |
| b. size/dimension/capacity: 1000 LBS/HR |  |  |  |
| c. blowers: |  | d. total flow rate: scfm |  |
| e. material stored/processed/handled: POLYSTYRENE FOAM SHEET |  |  |  |
| f. throughput/process rate/usage rate: |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 347406 |
| --- | --- | --- | --- |
| A. NAME: DART CONTAINER CORPORATION OF CALIFORNIA |  |  |  |
| B. ADDRESS: 150 S MAPLE ST. CITY: CORONA STATE: CA ZIP: 91720 |  |  |  |
| C. CONTACT PERSON: KIM KEENER |  | D. PHONE NO.: (909) 735-8115 |  |

| 4. PERMIT INFORMATION |  |  | App. No.: 347406 |
| --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  |  |  |
| B. AGENCY CONTACT PERSON: MARK LIU |  | C. PHONE NO.: (909) 396-2538 |  |
| D. PERMIT TO CONSTRUCT INFORMATION: P/C NO.: ISSUANCE DATE: 9/10/1999 |  |  |  |
| E. START-UP DATE: JAN 2000 |  |  |  |
| F. PERMIT TO OPERATE INFORMATION: P/O NO.: ISSUANCE DATE: |  |  |  |

| 5. EMISSION INFORMATION |  |  | App. No.: 347406 |
| --- | --- | --- | --- |
| A. PERMIT |  |  |  |
| A1. PERMIT LIMIT: (1) THE TOTAL QUANTITY OF MATERIAL PROCESSED IN THE EXTRUDER SHALL NOT EXCEED 24,000 POUNDS IN ANY ONE DAY, AND (2) THE TOTAL QUANTITY OF VOC EMISSIONS FROM THE FACILITY SHALL NOT EXCEED 3281 POUNDS IN ANY ONE DAY. |  |  |  |
| A2. bact/lAer DETERMINATION: AFTERBURNER FOR THE VOC EMISSIONSWITH A MINIMUM OF 98% DESTRUCTION EFFICIENCY |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |
| B1. MANUFACTURER/SUPPLIER: SMITH ENVIRONMENTAL |  |  |  |
| B2. TYPE: REGENERATIVE THERMAL OXIDIZER |  |  |  |
| B3. DESCRIPTION: 8 MMBTU/HOUR, NATURAL GAS FIRED, WITH TWO CERAMIC BEDS |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: 347394 ISSUANCE DATE: 9/10/99 P/O NO.: ISSUANCE DATE: |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: 35,000 cfm ACTUAL CONTAMINANT LOADING: aPPRX. 200 LBS/HR ISOPENTANE BLOWER HP: 250 HP |  |  |  |
| B6. WARRANTY: |  |  |  |
| B7. PRIMARY POLLUTANTS: VOC |  |  |  |
| B8. SECONDARY POLLUTANTS: COMBUSTION CONTAMINANTS |  |  |  |
| B9. SPACE REQUIREMENT: |  |  |  |
| B10. LIMITATIONS: |  |  |  |
| B11. LOCATION OF PRIOR DEMONSTRATION & AGENCY: FACILITY: CONTACT PERSON: PHONE NO.: AGENCY: ADDRESS: CONTACT PERSON: PHONE NO.: |  |  |  |
| B12. OPERATING HISTORY: IT WILL BE INSTALLED IN DECEMBER 1999 |  |  |  |
| B13. SOURCE TEST/PERFORMANCE DATA ANALYSIS: DATE OF SOURCE TEST: SOURCE TEST WILL BE PERFORMED SHORTLY AFTER INSTALLATION IS COMPLETED CAPTURE EFFICIENCY: DESTRUCTION EFFICIENCY: OVERALL EFFICEINCY: PERFORMANCE DATA: |  |  |  |
| B14. SOURCE TEST CONDITIONS/PERFORMANCE DATA: |  |  |  |
| C. COST |  |  |  |
| C1. CONTROL EQUIPMENT COST: check if installation cost is included in capital cost CAPITAL: $ INSTALLATION: $ ( ) SOURCE OF COST DATA: |  |  |  |
| C2. ANNUAL OPERATIONAL/MAINTENANCE COST: $ ( ) SOURCE OF COST DATA: |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: A SOURCE TEST IS REQUIRED |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: DATES: CAUSES: |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: DATES: CAUSES: |  |  |  |
| D5. FREQUENCY OF MAINTENANCE: |  |  |  |

| 6. COMMENTS |  | App. No.: 347406 |
| --- | --- | --- |
| THE APPLICANT RECEIVED PERMITS TO CONSTRUCT EIGHT DIRECT INJECTION EXTRUSION LINES (APPLICATION NUMBERS 347406-7, 347409, 347412-6). THE VOC EMISSIONS FROM THE EQUIPMENT WILL BE VENTED TO A THERMAL OXIDIZER (APPLICATION NUMBER 347394). |  |  |