Application No.:	  356145

Equipment Category – Polyurethane Tube Manufacturing

| 1. general information |  |  | DATE: 10/15/1999 |
| --- | --- | --- | --- |
| a. manufacturer: |  |  |  |
| b. type: Polyurethane Tube (Bio-Cannula) Manufacturing System |  | c. model: |  |
| d. style: |  |  |  |
| e. applicable aqmd regulation xi rules: |  |  |  |
| f. cost: $ (1999) source of cost data: |  |  |  |
| g. operating schedule: 16 hrs/day 7 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION | Other equipment |  | App. No.: 356145 |
| --- | --- | --- | --- |
| A. FUNCTION: To produce polyurethane tubes called bio-cannulas which are used as medical devices |  |  |  |
| b. size/dimension/capacity: |  |  |  |
| c. blowers: |  | d. total flow rate: scfm |  |
| e. material stored/processed/handled: Polyurethane pellets, tetrahydrofuran, isopropyl alcohol, mixture of 88% HFE-7100 with 12% IPA, HFE-7200, and methylethylketone. HFE-7100 and HFE-7200 are part of a class of compounds generally known as hydrofluoroethers and they are classified by the SCAQMD as Group I exempt compounds. |  |  |  |
| f. throughput/process rate/usage rate: |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 356145 |
| --- | --- | --- | --- |
| A. NAME: Medtronic Avalon Laboratories, Inc. |  |  |  |
| B. ADDRESS: 2610 E. Homestead Place CITY: Rancho Dominguez STATE: CA ZIP: 90220 |  |  |  |
| C. CONTACT PERSON: Gary Bernauer |  | D. PHONE NO.: (616)242-5259 |  |

| 4. PERMIT INFORMATION |  |  | App. No.: 356145 |
| --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  |  |  |
| B. AGENCY CONTACT PERSON: Belinda C. Wan |  | C. PHONE NO.: (909)396-2532 |  |
| D. PERMIT TO CONSTRUCT INFORMATION: P/C NO.: 356145 ISSUANCE DATE: 10/14/1999 |  |  |  |
| E. START-UP DATE: 12/1/1999 |  |  |  |
| F. PERMIT TO OPERATE INFORMATION: P/O NO.: ISSUANCE DATE: |  |  |  |

| 5. EMISSION INFORMATION |  |  | App. No.: 356145 |
| --- | --- | --- | --- |
| A. PERMIT |  |  |  |
| A1. PERMIT LIMIT: |  |  |  |
| A2. bact/lAer DETERMINATION: 4 non-regenerative carbon adsorbers, each containing 4000 lbs of activated carbon |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |
| B1. MANUFACTURER/SUPPLIER: American Filtration Systems, Inc., Model Boxsorber |  |  |  |
| B2. TYPE: carbon adsorber |  |  |  |
| B3. DESCRIPTION: Two Parallel Systems Consisting of Two Carbon Adsorbers in Series, American Filtration Systems, Inc., Model Boxsorber, With a Pre-Filter, Each Containing 4000 Lbs of Activated Carbon |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: 356146 ISSUANCE DATE: 10/14/1999 P/O NO.: ISSUANCE DATE: |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: ACTUAL CONTAMINANT LOADING: 62.7 lbs/day TOG BLOWER HP: HP |  |  |  |
| B6. WARRANTY: minimum overall control efficiency of 95% |  |  |  |
| B7. PRIMARY POLLUTANTS: VOC and exempt organic compounds such as ethoxy-nonafluorobutane |  |  |  |
| B8. SECONDARY POLLUTANTS: none |  |  |  |
| B9. SPACE REQUIREMENT: |  |  |  |
| B10. LIMITATIONS: The primary carbon adsorber beds shall be replaced with regenerated or fresh carbon before the accumulated total carbon spent reaches 7,350 pounds. The secondary carbon adsorber beds shall be replaced with regenerated or fresh carbon before the accumulated total carbon spent reaches 4,000 pounds. The total accumulated carbon spent is the sum of total carbon spent each operating day as determined by the following formula: Total Carbon Spent per Operating Day = 3.77 A + 6.42 B + 2.08 C + 1.49 D + 9.15 E where A = lb per day of tetrahydrofuran, B = lb per day of isopropyl alcohol, C = lb per day of mixture of 88% HFE-7100 and 12% IPA, D = lb per day of HFE-7200, and E = lb per day of methylethylketone. |  |  |  |
| B11. LOCATION OF PRIOR DEMONSTRATION & AGENCY: FACILITY: CONTACT PERSON: PHONE NO.: AGENCY: ADDRESS: CONTACT PERSON: PHONE NO.: |  |  |  |
| B12. OPERATING HISTORY: |  |  |  |
| B13. SOURCE TEST/PERFORMANCE DATA ANALYSIS: DATE OF SOURCE TEST: CAPTURE EFFICIENCY: DESTRUCTION EFFICIENCY: OVERALL EFFICEINCY: PERFORMANCE DATA: |  |  |  |
| B14. SOURCE TEST CONDITIONS/PERFORMANCE DATA: |  |  |  |
| C. COST |  |  |  |
| C1. CONTROL EQUIPMENT COST: check if installation cost is included in capital cost CAPITAL: $ INSTALLATION: $ (1999) SOURCE OF COST DATA: |  |  |  |
| C2. ANNUAL OPERATIONAL/MAINTENANCE COST: $ (1999) SOURCE OF COST DATA: |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: DATES: CAUSES: |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: DATES: CAUSES: |  |  |  |
| D5. FREQUENCY OF MAINTENANCE: |  |  |  |

| 6. COMMENTS |  | App. No.: 356145 |
| --- | --- | --- |
| The carbon adsorber controls the VOC emissions from both this equipment and the degreaser included in Application Number 356147. |  |  |