Application No.:	  358727

Equipment Category – Flow Coater, Dip Tank and Roller Coater

| 1. general information |  |  | DATE: 5/2/2002 |
| --- | --- | --- | --- |
| a. manufacturer: |  |  |  |
| b. type: Roller coater |  | c. model: |  |
| d. style: |  |  |  |
| e. applicable aqmd regulation xi rules: 1145, 1171 |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: 10 hrs/day 6 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 358727 |
| --- | --- | --- | --- |
| A. FUNCTION: Applies coating to silvered surface of glass as final step in mirror manufacture. Facility also has a curtain coater (A/N 358 728) and electric oven (A/N 358729). |  |  |  |
| b. size/dimension/capacity: 4 rollers, 2 troughs, 8.5' wide |  |  |  |
| c. blowers: 1.5 hp |  | d. total flow rate: scfm |  |
| e. material stored/processed/handled: Mirror backing material (Fenzi Extra Marrone) cut with thinner |  |  |  |
| f. throughput/process rate/usage rate: |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 358727 |  |
| --- | --- | --- | --- | --- |
| A. NAME: Maran-Wurzell Glass & Mirror |  |  |  | B. SIC CODE: 3231 |
| C. ADDRESS: 1683 Mount Vernon Ave. CITY: Pomona STATE: CA ZIP: 91766 |  |  |  |  |
| D. CONTACT PERSON: Jeff Wurzell |  | E. PHONE NO.: 800-223-4256 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: 358727 |
| --- | --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Kambiz Hadjforoosh |  |  | D. PHONE NO.: 909-396-3205 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: 358727 (A/N) ISSUANCE DATE: 12/28/1999 check if no p/c P/O NO.: F43711 ISSUANCE DATE: 9/4/2001 |  |  |  |  |
| F. START-UP DATE: Before Nov. 2000 |  |  |  |  |

| 5. EMISSION INFORMATION |  |  |  | App. No.: 358727 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |  |
| A1. PERMIT LIMIT: Maximum ethylene glycol butyl ether in coating material 10%, and maximum xylene in thinner 20%. Maximum VOC from facility 67 lb/day. Coaters and oven must be vented to conrol device. Associated control device (A/N 358730) is thermal oxidizer with permit requirement of 1600F minimum temperature and 90% overall control efficiency. |  |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: Rule 1145 (VOC content of coating materials not to exceed 500 g/l for curtain coating and 430 g/l for roller coating) and 1171 (VOC content of cleanup materials not to exceed 550 g/l) compliant materials. Coaters and oven vented to VOC control device with 90% overall (collection and destruction/removal) efficiency. |  |  |  |  |  |  |
| A3. BASIS OF THE BACT DETERMINATION: Offered by applicant. |  |  |  |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Airex |  |  |  |  |  |  |
| B2. TYPE: Regenerative thermal oxidizer, natural gas fired. |  |  |  |  |  |  |
| B3. DESCRIPTION: Has low-NOx burner. |  |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: 358730 ISSUANCE DATE: 12/28/1999 P/O NO.: F43714 ISSUANCE DATE: 9/4/2001 |  |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: 5400 dscfm (Typ.) ACTUAL CONTAMINANT LOADING: 4500 ppm as CH4 BLOWER HP: 1.5 |  |  |  |  |  |  |
| B6. WARRANTY: |  |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: VOC |  |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: NOx, CO, PM |  |  |  |  |  |  |
| B9. SPACE REQUIREMENT: |  |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  |  |  | B11. UNUSED |
| B12. OPERATING HISTORY: Thermal oxidizer has worked well since startup. |  |  |  |  |  |  |
| B13. UNUSED |  |  | B14. UNUSED |  |  |  |
| ### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in capital cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: |  |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: DATES: CAUSES: |  |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: DATES: CAUSES: |  |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |  |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: June 21-22, 2001 CAPTURE EFFICIENCY: 100% DESTRUCTION EFFICIENCY: 97.1% OVERALL EFFICEINCY: 97.1% SOURCE TEST/PERFORMANCE DATA: NOx: 1.1 ppm uncorrected and .03 lb/hr; CO: <4 ppm (uncorrected) OPERATING CONDITIONS: Normal TEST METHODS: Protocol approved by Monitoring & Source Test Engineering. Test report approved by AQMD permit enginer Khang Nguyen. |  |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: 358727 |
| --- | --- | --- |