Application No.:	  384630

Equipment Category – Tank Degassing System

| 1. general information |  |  | DATE: 3/1/2003 |
| --- | --- | --- | --- |
| a. manufacturer: |  |  |  |
| b. type: |  | c. model: |  |
| d. style: Vacuum line and blowers |  |  |  |
| e. applicable aqmd regulation xi rules: 1149 |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: 24 hrs/day 7 days/wk wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 384630 |
| --- | --- | --- | --- |
| A. FUNCTION: Portable tank degassing system. |  |  |  |
| b. size/dimension/capacity: 2500 scfm max. air pull rate. |  |  |  |
| c. blowers: |  | d. total flow rate: 2500 scfm |  |
| e. material stored/processed/handled: air plus tank vapors |  |  |  |
| f. throughput/process rate/usage rate: 2500 scfm max. |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 384630 |  |
| --- | --- | --- | --- | --- |
| A. NAME: Envent Corp. |  |  |  | B. SIC CODE: 8711 |
| C. ADDRESS: 2187 Walnut Ave. CITY: Signal Hill STATE: CA ZIP: 90806 |  |  |  |  |
| D. CONTACT PERSON: Thomas L. Kerscher |  | E. PHONE NO.: 562-997-9465 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: 384630 |
| --- | --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Hui Sung Choe |  |  | D. PHONE NO.: 909-396-2259 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: F39976 ISSUANCE DATE: 5/24/2001 check if no p/c P/O NO.: F39976 ISSUANCE DATE: 5/24/2001 |  |  |  |  |
| F. START-UP DATE: June 2001 |  |  |  |  |

| 5. EMISSION INFORMATION |  |  |  | App. No.: 384630 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |  |
| A1. PERMIT LIMIT: Restricted to degassing of tanks containing non-chlorinated petroleum hydrocarbon vapors, with exception of trace (<0.1 ppm) chlorinated hydrocarbons. VOC at outlet not to exceed 50 ppmv as hexane (measured hourly). Temperature at outlet of oxidizer to be at least 1400F in thermal mode, 600F in catalytic mode. Benzene at outlet not to exceed (ppmv limits based on distance, in meters, to nearest receptor): 25<50 .03, 50<75 .06, 75<100 0.11, 100<150 0.18, 150<200 0.28, 200<500 0.65, 500 or more 3.4. Minimum degassing time = 2.3 x V/Q, where V=tank volume and Q= volumetric suction rate (Rule 1149). |  |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: Permit limits on VOC and oxidizer temperatures |  |  |  |  |  |  |
| A3. BASIS OF THE BACT DETERMINATION: The VOC concentration limit is consistent with 5000 ppm max. vapor concentration in the tank and 99.9% destruction efficiency, with a factor of 10 margin. The 99.9% destruction efficiency was based on AQMD's knowledge of similer oxidizers used in soil vapor recovery systems. |  |  |  |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Envent |  |  |  |  |  |  |
| B2. TYPE: Thermal oxidizer and catalytic oxidizer, Model EMTOS 2500 |  |  |  |  |  |  |
| B3. DESCRIPTION: Natural gas or LPG fired with EPCON Model 3-DF-2500-H-T Low Nox Burner |  |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: F39976 ISSUANCE DATE: 5/24/2001 P/O NO.: F39976 ISSUANCE DATE: 5/24/2001 |  |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: 2500 scfm ACTUAL CONTAMINANT LOADING: BLOWER HP: |  |  |  |  |  |  |
| B6. WARRANTY: |  |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: VOC |  |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: NOx, CO |  |  |  |  |  |  |
| B9. SPACE REQUIREMENT: |  |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  |  |  | B11. UNUSED |
| B12. OPERATING HISTORY: The owner reports that the system has been used on 5 or 6 tanks to date, and the 50 ppmv VOC limit has been met in all cases. |  |  |  |  |  |  |
| B13. UNUSED |  |  | B14. UNUSED |  |  |  |
| ### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in capital cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: |  |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: 0 DATES: CAUSES: |  |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: None since this P/C date DATES: CAUSES: |  |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |  |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: CAPTURE EFFICIENCY: DESTRUCTION EFFICIENCY: OVERALL EFFICEINCY: SOURCE TEST/PERFORMANCE DATA: OPERATING CONDITIONS: TEST METHODS: |  |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: 384630 |
| --- | --- | --- |
| TBACT was considered to be use of a thermal oxidizer. The original date of this listing was 12/18/01. An administrative change (A/N 405426, AQMD Permit reissued 9/6/2002) was added 3/1/2003, changing name of oxidizer manufacturer from EPCON to ENVENT.. |  |  |