Application No.:	  392961

Equipment Category – Tunnel Washer

| 1. general information |  |  | DATE: 8/21/2003 |
| --- | --- | --- | --- |
| a. manufacturer: Milnor |  |  |  |
| b. type: Counterflow batch |  | c. model: 76032CBW |  |
| d. style: Top-transfer, 10-chamber |  |  |  |
| e. applicable aqmd rules: 212, 401, 402, 1303, 1401 |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: 20 hrs/day 7 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 392961 |
| --- | --- | --- | --- |
| A. FUNCTION: Extracts VOC from shop towels by steam and hot water wash. Towel batches are transferred from chamber to chamber in series. Fresh water is added to the last two chambers. The wash water is pumped from chamber to chamber in general counter-flow to the towels but is also sewered from some chambers while being recirculated from some chambers to other chambers. The first six modules are used to steam strip volatiles out of the towels, and very little water is used in these modules. Soap may be used in the second and fifth modules. The last four modules are used for hot water rinse |  |  |  |
| b. size/dimension/capacity: 110 lb per batch |  |  |  |
| c. blowers: |  | d. total flow rate: scfm |  |
| e. material stored/processed/handled: VOC-laden shop towels |  |  |  |
| f. throughput/process rate/usage rate: 80,000 lb per week (dry basis) |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 392961 |  |
| --- | --- | --- | --- | --- |
| A. NAME: Textek, Inc. |  |  |  | B. SIC CODE: 7218 |
| C. ADDRESS: 45585 Commerce Street CITY: Indio STATE: CA ZIP: 92201 |  |  |  |  |
| D. CONTACT PERSON: Lola Milburn |  | E. PHONE NO.: 760-347-3511 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: 392961 |
| --- | --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Hemang Desai |  |  | D. PHONE NO.: 909-396-2596 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: 392961 ISSUANCE DATE: 2/13/2002 check if no p/c P/O NO.: F61401 ISSUANCE DATE: 6/13/2003 |  |  |  |  |
| F. START-UP DATE: May 2002 |  |  |  |  |

| 5. EMISSION INFORMATION |  |  |  | App. No.: 392961 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |  |
| A1. PERMIT LIMIT: Facility monthly VOC emission limit of 667 lb. Daily processing limit of 48,000 lb towels. Washer must be vented to primary and secondary condensers followed by afterburner. The temperature in the afterburner must be at least 1400F, and overall VOC control efficiency of the afterburner must be at least 90%. |  |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: Condenser followed by afterburner with the latter achieving 90% overall VOC control efficiency, which was offered by the applicant, is now achieved in practice for this equipment category. |  |  |  |  |  |  |
| A3. BASIS OF THE BACT DETERMINATION: Achieved in practice. |  |  |  |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Reliable Thermal Technologies |  |  |  |  |  |  |
| B2. TYPE: Afterburner, recuperative type |  |  |  |  |  |  |
| B3. DESCRIPTION: The afterburner system (model 8M50X) includes a heat exchanger to preheat incoming VOC-laden gases using sensible heat from the hot exhaust. It is rated at 10 MMBtu/hr maximum input. Prior to reaching the afterburner, the VOC-laden gases pass through condensers, which have an optional chilled water system. The vents from the first three or four washing chambers go to an auxilliary condenser then to the main condenser. The vents from the remaining six or seven chambers go directly to the main condenser. There is a sump for the dirty wash water, which also vents to the main condenser. The dirty towel loading area is enclosed, and the enclosure is vented to the oxidizer. |  |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: 399831 ISSUANCE DATE: 4/15/2002 P/O NO.: F61402 ISSUANCE DATE: 6/13/2003 |  |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: 8,000 scfm ACTUAL CONTAMINANT LOADING: 1.5% VOC as CH4 BLOWER HP: 40 |  |  |  |  |  |  |
| B6. WARRANTY: The facility owner reports that the oxidizer supplier guranteed a minimum of 99.5% destruction efficiency. |  |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: VOC |  |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: NOx, CO, PM |  |  |  |  |  |  |
| B9. SPACE REQUIREMENT: 6'-4"W x 21'L x 14'-2"H |  |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  |  |  | B11. UNUSED |
| B12. OPERATING HISTORY: The afterburner system started up in May 2002. The afterburner was tested in October 2002 but failed to meet the 90% overall control efficiency. It was then adjusted and re-tested in February 2003 and exceeded the required control efficiency by a wide margin. Tha facility reports that the system has operated essentially trouble-free since the second source test. |  |  |  |  |  |  |
| B13. UNUSED |  |  | B14. UNUSED |  |  |  |
| ### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in capital cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: Hemang Desai INSPECTOR'S NAME: Dan Kasper DATE: 10/22/02 and 2/18/02 |  |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: Inspections and source tests |  |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: None DATES: CAUSES: |  |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: 1 DATES: 10/22/2002 CAUSES: Afterburner destruction efficiency was found to be less than the minimum 90% required in the Permit to Operate. |  |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |  |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: 2/18/2003 CAPTURE EFFICIENCY: 100% DESTRUCTION EFFICIENCY: 99.3% OVERALL EFFICIENCY: 99.3% SOURCE TEST/PERFORMANCE DATA: Inlet Inlet Exhaust Gas Flow, dscfm 3614 3302 O2, % (dry) 20.7 14.2 VOC, ppmvd as CH4 15,250 85 VOC mass flow, lb/hr as C 104.7 0.58 VOC removal, % 99.3 OPERATING CONDITIONS: TEST METHODS: U.S.EPA Method 204 was used to certify the dirty towel loading area as a permanent total enclosure. AQMD Methods 25.1 and 25.3 were used for inlet and outlet HC measurements.. The source test was accepted by AQMD's Monitoring and Source Test Engineering group. |  |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: 392961 |
| --- | --- | --- |
| VOC from the condensate should be vented to the oxidizer or otherwise controlled. |  |  |