- Application No.:  N-2253-15-2

- Equipment Category – Dryer or Oven

| 1. GENERAL INFORMATION |  |  | DATE: 4/16/2003 |
| --- | --- | --- | --- |
| a. manufacturer: Young Bros. |  |  |  |
| b. type: |  | c. model: Kelgraf |  |
| d. style: Sheet metal construction, refractory-lined |  |  |  |
| e. applicable aqmd rules: |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: 24 hrs/day 7 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: N-2253-15-2 |
| --- | --- | --- | --- |
| A. FUNCTION: Drying of coating applied to metal sheets used in can manufacture. This oven is in Line No.2 of three coating lines each with its own oven. The three ovens are identical. |  |  |  |
| B. maximum Heat Input: 8.6 MMBtu/hr |  | C. maximum throughput: 80 sheets/min. |  |
| D. burner information: no.: 5 type: Low-NOx |  |  |  |
| e. primary fuel: Natural Gas |  | f. other fuel: None |  |
| g. operating conditions: Steady |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: N-2253-15-2 |  |
| --- | --- | --- | --- | --- |
| A. NAME: Ball Western Can Co. |  |  |  | B. SIC CODE: 3411 |
| C. ADDRESS: 300 W. Gregor Street CITY: Oakdale STATE: CA ZIP: 95361 |  |  |  |  |
| D. CONTACT PERSON: Bob Hall |  | E. PHONE NO.: 303-460-5445 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: N-2253-15-2 |
| --- | --- | --- | --- | --- |
| A. AGENCY: SJVAPCD |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Mark Schonhoff |  |  | D. PHONE NO.: 209-557-6448 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: N-2253-15-2 ISSUANCE DATE: 10/7/2001 check if no p/c P/O NO.: ISSUANCE DATE: |  |  |  |  |
| F. START-UP DATE: June 2001 |  |  |  |  |

| 5. EMISSION INFORMATION |  |  | App. No.: N-2253-15-2 |  |  |
| --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |
| A1. PERMIT LIMIT: Maximum emissions from oven as lb/MMBtu: NOx-.024 (20 ppmvd@3%O2), CO-0.15, SOx-.00214, PM10-.0076, VOC from gas combustion-.005. Oven to be vented to thermal oxidizer. Maximum emissions from thermal oxidizer as lb/MMBtu: NOx-.098, CO-.082, VOC-.005, SOx-.00214, PM10-.0076. Facility VOC annual and quarterly caps. Thermal oxidizer temperature to be maintained at or above value determined in source test to provide 98.5 overall VOC control. |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: 20 ppmvd NOx @3%O2 (low-NOx burner), 95% overall VOC control (capture and incineration) |  |  |  |  |  |
| a3. BASIS OF THE BACT/LAER DETERMINATION: BACT determination by San Joaquin Valley Unified APCD. |  |  |  |  |  |
| ##### B. CONTROL TECHNOLOGY |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Maxon |  |  |  |  |  |
| B2. TYPE: Low-NOx Burner |  |  |  |  |  |
| B3. DESCRIPTION: Cyclomax |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: N-2253-15-2 ISSUANCE DATE: 10/7/2001 P/O NO.: ISSUANCE DATE: |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: ACTUAL CONTAMINANT LOADING: BLOWER HP: |  |  |  |  |  |
| B6. WARRANTY: Burners are guaranteed not to exceed .024 lb/MMBtu NOx |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: NOx, CO, VOC, PM10, SOx |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: |  |  |  |  |  |
| B9. SPACE REQUIREMENT: |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  | ###### B11. UNUSED |  |
| B12. OPERATING HISTORY: Ovens have been in regular use almost every day since startup. |  |  |  |  |  |
| ### B13. UNUSED |  | ### B14. UNUSED |  |  |  |
| ##### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in EQUIPMENT cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: Initial source test for NOx concentration from the oven and overall VOC control efficiency. Annual source test for thermal oxidizer VOC destruction efficiency. |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: DATES: CAUSES: |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: DATES: CAUSES: |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: 2/9/2002 CAPTURE EFFICIENCY: DESTRUCTION EFFICIENCY: OVERALL EFFICIENCY: SOURCE TEST/PERFORMANCE DATA: Oven MMBtu/hr Input % of Max. Raw NOx, ppmvd O2, % (dry) NOx@3%O2 2 3.1 36.1 1.1 19.3 12.3 3 3.3 38.7 0.6 18.8 5.1 4 2.9 34.1 0.9 18.8 7.7 OPERATING CONDITIONS: TEST METHODS: Each test was an average of three 30-minute tests. Tests were approved by SJVAPCD. |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: N-2253-15-2 |
| --- | --- | --- |
| Burner sizes used on these ovens are 0.8, 1.6 and 5.4 MMBtu/hr. Oven temperature is normally about 325 to 380 F. These burners have turndown capability of 15:1. Different requirements in other cases may affect the manufacturer's emissions guarantee. |  |  |