Application No.:	  06-320-001B

Equipment Category - Rotogravure Printing

| 1. general information |  |  | DATE: 11/7/2000 |
| --- | --- | --- | --- |
| a. manufacturer: Fuji Kikai Kogyo Company, Ltd. |  |  |  |
| b. type: Product and packaging rotogravures |  | c. model: |  |
| d. style: |  |  |  |
| e. applicable aqmd regulation xi rules: |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: 24 hrs/day 7 days/wk 50 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 06-320-001B |
| --- | --- | --- | --- |
| A. FUNCTION: The presses are used to print decorative paper for laminations which are manufactured into kitchen and bath counter tops, wood paneling, kitchen cabinets, ready-to-assemble furniture and other products. |  |  |  |
| b. size/dimension/capacity: 1) Three seven-color-station presses that use interchangeably solvent base inks and water base inks; and 2) one four-color-station press that only uses water base inks. |  |  |  |
| c. blowers: |  | d. total flow rate: scfm |  |
| e. material stored/processed/handled: |  |  |  |
| f. throughput/process rate/usage rate: |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 06-320-001B |  |
| --- | --- | --- | --- | --- |
| A. NAME: Chiyoda America, Inc. |  |  |  | B. SIC CODE: 2754 |
| C. ADDRESS: P.O. Box 470 CITY: Morgantown STATE: PA ZIP: 19543-0470 |  |  |  |  |
| D. CONTACT PERSON: John Kotrich |  | E. PHONE NO.: (610) 386-3195 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: 06-320-001B |
| --- | --- | --- | --- | --- |
| A. AGENCY: Pennsylvania Department of Environmental Protection |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Roger A. Fitterling |  |  | D. PHONE NO.: (610) 916-0100 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: various P/Cs ISSUANCE DATE: 1988-1995 check if no p/c P/O NO.: 06-320-001B ISSUANCE DATE: 08/02/99 |  |  |  |  |
| F. START-UP DATE: The first two presses in 1988; the third in 1992; and the fourth in 1995 |  |  |  |  |

| 5. EMISSION INFORMATION |  |  |  | App. No.: 06-320-001B |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |  |
| A1. PERMIT LIMIT: 1) Facility's VOC emissions shall not exceed 1000 lbs/day and/or 49 tons during any consecutive 12-month period; 2) Combustion temperatures of thermal oxidizers shall be maintained at a mimimum of 1500 degree F (a surrogate measurement for 99% destruction efficiency); 3) Enclosure shall be maintained to ensure 100% capture efficiency. |  |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: Enclosure and thermal oxidizers at 99% overall control efficiency. |  |  |  |  |  |  |
| A3. BASIS OF THE BACT DETERMINATION: Proposed by applicant |  |  |  |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Smith Engineering Company and Salem Engelhard |  |  |  |  |  |  |
| B2. TYPE: |  |  |  |  |  |  |
| B3. DESCRIPTION: Emissions from four enclosed presses are vented to two oxidizers having a common stack: 1) Thermal oxidizer manufactured by Smith Engineering Company consists of a combustion chamber and three heat exchangers for heat recovery; 38 mmbtu/hr natural gas fired; and operated at a minimum combustion temperature of 1500 degree F and approximately 1.5 to 2.5 sec retention time. 2) Regenerative thermal oxidizer manufactured by Salem Engelhard, 32 mmbtu/hr natural gas fired, and operated at a minimum combustion temperature of 1500 degree F. |  |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: 06-320-001A&B ISSUANCE DATE: 1990&1995 P/O NO.: 06-320-001B ISSUANCE DATE: 8/2/99 |  |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: ACTUAL CONTAMINANT LOADING: BLOWER HP: |  |  |  |  |  |  |
| B6. WARRANTY: |  |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: VOC |  |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: |  |  |  |  |  |  |
| B9. SPACE REQUIREMENT: |  |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  |  |  | B11. UNUSED |
| B12. OPERATING HISTORY: |  |  |  |  |  |  |
| B13. UNUSED |  |  | B14. UNUSED |  |  |  |
| ### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in capital cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: Record Keeping and Source Testing |  |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: DATES: CAUSES: |  |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: DATES: CAUSES: |  |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |  |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: See below CAPTURE EFFICIENCY: 100% DESTRUCTION EFFICIENCY: 99% OVERALL EFFICEINCY: 99% SOURCE TEST/PERFORMANCE DATA: --- On 10/1/99, a test was performed to measure destruction efficiency of Salem Engelhard regenerative thermal oxidizer. The results were: RUN#1 - Inlet: 55,500 dscfm, 2253 ppmv (854.14 lb/hr) VOC measured as propane. Outlet: 63,600 dscfm, 20 ppmv (8.74 lb/hr) VOC. RUN#2 - Inlet: 56,300 dscfm, 2180 ppmv (839.14 lb/hr) VOC. Outlet: 62,700 dscfm, 20 ppmv (8.38 lb/hr) VOC. RUN#3 - Inlet: 56,100 dscfm, 2156 ppmv (834.43 lb/hr) VOC. Outlet: 63,600 dscfm, 19 ppmv (8.45 lb/hr) VOC. Destruction efficiency was 99.0%. --- On 6/30/92, a test was performed to measure destruction efficiency of Smith Engineering Company thermal oxidizer. The averages of three test runs were: Inlet: 109,292 dscfm, 3,446 ppmv (701.23 lb/hr) VOC measured as carbon. Outlet: 106,231 dscfm, 28 ppmv (5.54 lb/hr) VOC. Destruction efficiency was 99.2%. OPERATING CONDITIONS: TEST METHODS: EPA Test Method 25A and EPA Method 204 |  |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: 06-320-001B |
| --- | --- | --- |
| Permit to construct was first issued to two new presses at Advanced Printing Technology, Inc. in 1988. At that time, BACT standards for rotogravure presses were 60% capture efficiency and 90% destruction efficiency. The company proposed to control emissions from these presses by using a Smith thermal oxidizer and an enclosure. The enclosure was designed to meet EPA draft criteria for 100% capture efficiency. A first source test conducted in 1989 showed that 99% efficiency would be obtained by maintaining a minimum combustion temperature of 1500 degree F. The facility was not required to provide offsets by operating the equipment under a facility's VOC cap of 1000 lbs/day or 49 tons/yr. After 1988, change of ownership occurred, two more presses and a Salem Engelhard thermal oxidizer were added, and two more source source tests were conducted to assess the efficiency of the oxidizers. The facility cap remains at 1000 lbs/day or 49 tons/yr. |  |  |