Application No.:	  R14-12B

Equipment Category – Rotogravure Printing

| 1. general information |  |  | DATE: 12/2/2003 |
| --- | --- | --- | --- |
| a. manufacturer: Cerruti |  |  |  |
| b. type: Publication, four-color |  | c. model: |  |
| d. style: |  |  |  |
| e. applicable aqmd rules: |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: hrs/day days/wk wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: R14-12B |
| --- | --- | --- | --- |
| A. FUNCTION: Facility prints magazines, catalogs and advertising materials. |  |  |  |
| b. size/dimension/capacity: Eighteen presses: ten 108" 8-unit presses, two 108" 12-unit presses, six 133" 8-unit presses |  |  |  |
| c. blowers: |  | d. total flow rate: scfm |  |
| e. material stored/processed/handled: |  |  |  |
| f. throughput/process rate/usage rate: |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: R14-12B |  |
| --- | --- | --- | --- | --- |
| A. NAME: Quad/Graphics, Inc. |  |  |  | B. SIC CODE: 2754 |
| C. ADDRESS: 855 Caperton Blvd. CITY: Martinsburg STATE: WV ZIP: 25401 |  |  |  |  |
| D. CONTACT PERSON: Tony Lewis |  | E. PHONE NO.: 304-260-7167 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: R14-12B |
| --- | --- | --- | --- | --- |
| A. AGENCY: West Virginia DEP |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Joe Kessler |  |  | D. PHONE NO.: 304-926-3727 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: R14-12B ISSUANCE DATE: 9/14/1995 check if no p/c P/O NO.: R14-12B ISSUANCE DATE: 8/30/2001 |  |  |  |  |
| F. START-UP DATE: The first two presses went into operation in 1996 or 1997. |  |  |  |  |

| 5. EMISSION INFORMATION |  |  |  | App. No.: R14-12B |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |  |
| A1. PERMIT LIMIT: Title V permit. Presses are to be enclosed in a permanent total enclosure vented to a solvent recovery system (SRS) with minimum 98.25% VOC removal efficiency. A new SRS module must achieve 92% VOC removal efficiency for each calendar month during its first six months of operation. SRS removal efficiency must be monitored by either continuous monitoring or periodic source testing. Ink and solvent tanks must have vapor balance systems to prevent VOC release or be vented to the SRS. Tank fill rates must not exceed 80 gpm. Facility records must demonstrate a minimum 95% overall VOC control for each calendar month and a minimum 96% overall VOC control for every 12-month period. Presses, tanks, SRS stacks, solvent load-out operations and a cylinder washer have individual hourly and 12-monthly VOC mass emission limits. |  |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: Enclosure of presses in permanent total enclosure vented to SRS with minimum 98.25% VOC removal efficiency and facility-wide control of VOC at 95% for each calendar month and 96% for each 12-month period. |  |  |  |  |  |  |
| A3. BASIS OF THE BACT DETERMINATION: This BACT determination was made after seven presses had been installed and operated and was based on VOC control that had been achieved while operating those presses. |  |  |  |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Barneby & Sutcliffe |  |  |  |  |  |  |
| B2. TYPE: Carbon adsorption |  |  |  |  |  |  |
| B3. DESCRIPTION: Three banks of seven adsorption modules with one stack per bank. Modules are regenerated on a rotating basis using steam. The steam/VOC mixture passes to a condenser/separator system, where recovered solvent is returned to the process. Uncondensed VOC is vented to the SRS. Some recovered solvent is removed from the facility. The solvent is essentially all toluene, with traces of xylene and ethylbenzene. Boilers producing steam are restricted to natural gas or LPG, are required to be equipped with low-NOx burners and to use flue gas recirculation for NOx control, and are subject to hourly and 12-monthly restrictions on mass emissions of NOx, CO, VOC, PM10 and SO2. |  |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: R14-12B ISSUANCE DATE: 9/14/1995 P/O NO.: R14-12B ISSUANCE DATE: 8/30/2001 |  |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: 400,000 acfm (design) ACTUAL CONTAMINANT LOADING: 1400 ppm as toluene (typ.) BLOWER HP: |  |  |  |  |  |  |
| B6. WARRANTY: |  |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: VOC |  |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: NOx, CO, PM10 |  |  |  |  |  |  |
| B9. SPACE REQUIREMENT: |  |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  |  |  | B11. UNUSED |
| B12. OPERATING HISTORY: This is a long-term project, still in progress, with presses going into operation as they are added. First two presses went on line in 1996 or 1997. Seven presses were in operation as of the 2001 BACT determination. Eleven presses now in operation. The facility reports that it has had no problems complying with the permit conditions except for two months in which the 95% VOC control limit was not achieved due to equipment problems. |  |  |  |  |  |  |
| B13. UNUSED |  |  | B14. UNUSED |  |  |  |
| ### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in capital cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: Records of VOC usage and recovery with monthly reports; continuous monitoring of VOC removal by the SRS. |  |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: None DATES: CAUSES: |  |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: 2 DATES: NOV Oct 2002, NOD more recent CAUSES: Recordkeeping and reporting deficiencies |  |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |  |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: CAPTURE EFFICIENCY: DESTRUCTION EFFICIENCY: OVERALL EFFICIENCY: SOURCE TEST/PERFORMANCE DATA: OPERATING CONDITIONS: TEST METHODS: |  |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: R14-12B |
| --- | --- | --- |
| . |  |  |