| Source Activity | Mitigation Measure | PM10 Control Efficiency | Comments | Estimated Cost |
| --- | --- | --- | --- | --- |
| Active demolition and debris removal | Apply water every 4 hours to the area within 100 feet of a structure being demolished, to reduce vehicle trackout. | 36% |  | ND |
| Trackout | Use a gravel apron, 25 feet long by road width, to reduce mud/dirt trackout from unpaved truck exit routes. | 46% |  | $1,360/year (gravel apron dimensions: 50’ x 30’ x 3” thick) |
| Post-demolition stabilization | Apply dust suppressants (e.g., polymer emulsion) to disturbed areas upon completion of demolition. | 84% | For actively disturbed areas. | $5,340/acre-year (Useful life of 1 year) |
| Demolition Activities | Apply water to disturbed soils after demolition is completed or at the end of each day of cleanup. | 10% | 14-hour watering interval. | $68-$81/acre-day |
| Demolition Activities | Prohibit demolition activities when wind speeds exceed 25 mph. | 98% | Estimated for high wind days in absence of soil disturbance activities. Demolition of 1,000 ft2 structure on 1.2 acres. | $1.36 per 8 hour day idled |
| Construction Activities | Apply water every 3 hours to disturbed areas within a construction site. | 61% | 3.2-hour watering interval. | ND |
| Scraper loading and unloading | Require minimum soil moisture of 12% for earthmoving by use of a moveable sprinkler system or a water truck. Moisture content can be verified by lab sample or moisture probe. | 69% | AP-42 emission factor equation for materials handling due to increasing soil moisture from 1.4% to 12%. | $138/acre (sprinkler system to maintain minimum soil moisture of 12%) |
| Construction traffic | Limit on-site vehicle speeds (on unpaved roads) to 15 mph by radar enforcement. | 57% | Assume linear relationship between PM10 emissions and uncontrolled vehicle speed of 35 mph. | $22/inspection $180/sign |
| Source Component | Mitigation Measure | PM10 Control Efficiency | References & Assumptions | Estimated Cost |
| Grading | Replace ground cover in disturbed areas as quickly as possible. | 5% | EPA, “Control of Fugitive Dust Sources” EPA-450/3-88-008, September 1988 | ND |
| Grading | All trucks hauling dirt, sand, soil, or other loose materials are to be tarped with a fabric cover and maintain a freeboard height of 12 inches. | 91% | Arizona Department of Transportation Construction Analysis Services, “Final Field Study Report - PM10 Control Management Study for ADOT Construction Projects, June 1994 | ND |

Unless otherwise noted, information presented in this table is from the following reference:  WRAP Fugitive Dust Handbook, September 7, 2006 (http://www.wrapair.org/forums/dejf/fdh/content/FDHandbook_Rev_06.pdf). ND = No Data.

2003 dollars.

Unless otherwise noted, information presented in this table is from the following reference:  WRAP Fugitive Dust Handbook, September 7, 2006 (http://www.wrapair.org/forums/dejf/fdh/content/FDHandbook_Rev_06.pdf). ND = No Data.

2003 dollars.

Higher than 5% control efficiency may be used. However, please provide the supporting analysis and data in the environmental documentation.