| Source Activity | Mitigation Measure | PM10 Control Efficiency | Comments | Estimated Cost |
| --- | --- | --- | --- | --- |
| Conveyors | Continuous water spray at conveyor transfer point | 62% | The control efficiency achieved by increasing the moisture content of the material from 1% to 2% is calculated utilizing the AP-42 emission factor equation for materials handling which contains a correction term for moisture content. | ND |
| Storage piles | Require construction of 3-sided enclosures with 50% porosity for storage pile. | 75% | Determined through modeling of open area windblown emissions with 50% reduction in wind speed and assuming no emission reduction when winds approach open side. | $109/year (useful life of 15 years; pile volume = 5 yd3) |
| Storage piles | Water the storage pile by hand at a rate of 1.4 gallons/hour-yard2, or apply cover when wind events are declared. | 90% |  | ND |

Unless otherwise noted, information presented in this table is from the following reference:  WRAP Fugitive Dust Handbook, September 7, 2006 (http://www.wrapair.org/forums/dejf/fdh/content/FDHandbook_Rev_06.pdf). ND = No Data.

2003 dollars.