| Source Activity | Mitigation Measure | PM10 Control Efficiency | Comments | Estimated Cost |
| --- | --- | --- | --- | --- |
| Local streets | Implement street sweeping program with Rule 1186 compliant PM10 efficient vacuum units (14-day frequency) | 16% | For PM10 efficient sweepers, based on 86% efficient sweeping, 8.6 day return time, and CA-VMT weighted sweeping frequency (7 to 30 days) | Purchase PM10 efficient sweeper: $190/mile-year (useful life of 8 years; sweep 15 centerline miles per day) |
| Arterial/collector streets |  | 26% |  |  |
| Local, arterial and collector streets | Require streets to be swept by Rule 1186 compliant PM10 efficient vacuum units (once per month frequency) | 9% | For PM10 efficient sweepers, based on 86% efficient sweeping, 8.6 day return time, and CA-VMT weighted sweeping frequency (7 to 30 days) | Purchase PM10 efficient sweeper: $190/mile-year (useful life of 8 years; sweep 15 centerline miles per day) |
| Mud/dirt trackout | Install pipe-grid trackout-control device to reduce mud/dirt trackout from unpaved truck exit routes. | 80% |  | $1,820/year (useful life of 8 years) |
| Mud/dirt trackout | Install gravel bed trackout apron (3 inches deep, 25 feet long, 12 feet wide per lane and edged by rock berm or row of stakes) to reduce mud/dirt trackout from unpaved truck exit routes. | 46% |  | $1,360/year (50’ x 30’ x 3” thick) |
| Mud/dirt trackout | Require paved interior roads to be 100 feet long, 12 feet wide per lane and edged by rock berm or row of stakes, or add 4 foot shoulder for paved roads. | 42% |  | 4’ Paved Shoulders: $8,200/mile-year (useful life of 20 years) |

Unless otherwise noted, information presented in this table is from the following reference:  WRAP Fugitive Dust Handbook, September 7, 2006 (http://www.wrapair.org/forums/dejf/fdh/content/FDHandbook_Rev_06.pdf). ND = No Data.

2003 dollars.