| Source Activity | Mitigation Measure | PM10 Control Efficiency | Comments | Estimated Cost |
| --- | --- | --- | --- | --- |
| Storage pile wind erosion | Require construction of 3-sided enclosures with 50% porosity. | 75% | Determined through modeling of open area windblown emissions with 50% reduction in wind speed and assuming no emission reduction when winds approach open side | $109/year (Useful life of 15 years; pile volume = 5 yd3) |
| Storage pile wind erosion | Water the storage pile by hand or apply cover when wind events are declared. | 90% |  | $22/day (100 cubic yard pile) |
| Windblown dust from inactive areas | Apply chemical soil stabilizers on inactive construction areas (disturbed lands within construction projects that are unused for at least four consecutive days). | Up to 80% | Wind erosion from inactive areas. | ND |
| Windblown dust from disturbed areas | Plant tree windbreaks on the windward perimeter of construction projects if adjacent to open land. | 4% (15% for mature trees) |  | ND |
| Windblown dust from disturbed areas4 | Plant vegetative ground cover in disturbed areas as soon as possible. | 15% |  | ND |

Unless otherwise noted, information presented in this table is from the following reference:  WRAP Fugitive Dust Handbook, September 7, 2006 (http://www.wrapair.org/forums/dejf/fdh/content/FDHandbook_Rev_06.pdf). ND = No Data.

2003 dollars.

Reference: U.S. EPA, "AP-42, Vol. I." Pg. 11.2.4-1 (http://www.epa.gov/ttn/chief/old/ap42/4th_edition/ap42_4thed_withsuppsa_f.pdf).

Reference: SCAQMD, SIP for PM10 in the Coachella Valley, 1990, Pg 5-15.